|  | MOne MOne UAG API Reference (SMS, SMPP34) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Issue | 01 |  |  |
|  | Date | 2016-09-20 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

MOne V100R002C01 UAG API Reference (SMS, SMPP34)

Doc Number:OFFE00028460_PMD301ZH
Revision:A

拟制/Prepared by:  zhangyonghong WX368662;

评审/Reviewed by:

批准/Approved by:

2017-02-23

Huawei Technologies Co., Ltd. 
华为技术有限公司
All rights reserved
版权所有  侵权必究

Copyright © Huawei Technologies Co., Ltd. 2016. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | http://www.huawei.com |
| Email: | support@huawei.com |

Change History

| Issue | Date | Description |
| --- | --- | --- |
| 01 | 2016-09-20 | - First release for MOne V100R002C00 version. - Update the document style. |

Contents

Change History	ii

1 Overview	1

1.1 API Functions	1

1.2 Level of Requirement for Parameters	3

1.3 API Parameter Types	4

1.4 SMPP PDU Format	5

1.5 Mapping Between SMPP PDUs and APIs	10

1.6 Optional Parameter Format	10

1.7 Error Response Format	12

2 APIs for Receiving SMS Messages	17

2.1 Process	17

2.2 bind_receiver	19

2.2.1 Function	19

2.2.2 Request	19

2.2.3 Response	23

2.2.4 Error Codes	26

2.3 bind_transceiver	26

2.3.1 Function	26

2.3.2 Request	26

2.3.3 Response	30

2.3.4 Error Codes	33

2.4 enquire_link	33

2.4.1 Function	33

2.4.2 Request	33

2.4.3 Response	35

2.4.4 Error Codes	36

2.5 deliver_sm	36

2.5.1 Function	36

2.5.2 Request	36

2.5.3 Response	60

2.5.4 Error Codes	61

2.6 unbind	62

2.6.1 Function	62

2.6.2 Request	62

2.6.3 Response	63

2.6.4 Error Codes	64

3 APIs for Sending SMS Messages	65

3.1 Process	65

3.2 bind_transmitter	67

3.2.1 Function	67

3.2.2 Request	67

3.2.3 Response	72

3.2.4 Error Codes	74

3.3 submit_sm	74

3.3.1 Function	74

3.3.2 Request	75

3.3.3 Response	106

3.3.4 Error Codes	108

3.4 submit_multi	109

3.4.1 Function	109

3.4.2 Request	109

3.4.3 Response	143

3.4.4 Error Codes	145

3.5 deliver_sm(report)	146

3.5.1 Function	146

3.5.2 Request	146

3.5.3 Response	173

3.5.4 Error Codes	174

3.6 unbind	174

3.6.1 Function	174

3.6.2 Request	174

3.6.3 Response	176

3.6.4 Error Codes	177

Figures

Figure 2-1 Process of receiving SMS messages in Notify mode#fig	18

Figure 3-1 Process of sending SMS messages and receiving status reports in Notify mode#fig	66

Tables

Table 1-1 Functions of SMS capability APIs	1

Table 1-2 Level of requirement for parameters	3

Table 1-3 API parameter types	4

Table 1-4 Parameter length	5

Table 1-5 SMPP PDU format overview	5

Table 1-6 SMPP PDU format description	6

Table 1-7 Mapping between PDUs and APIs#table	10

Table 1-8 SMPP PDU Format Description	10

Table 1-9 Optional or Conditional Parameter Tag values	11

Table 1-10 GENERIC_NACK PDU Format Description	12

Table 1-11 SMPP34 return codes#table	13

Table 2-1 Description for the process of receiving SMS messages	18

Table 2-2 Parameters in a bind_receiver request message header	19

Table 2-3 Parameters in a bind_receiver request message body	20

Table 2-4 Parameters in a bind_receiver response message header	23

Table 2-5 Parameters in a response message body	25

Table 2-6 bind_receiver error codes	26

Table 2-7 Parameters in a bind_transceiver request message header	27

Table 2-8 Parameters in a bind_transceiver request message body	28

Table 2-9 Parameters in a bind_transceiver response message header	31

Table 2-10 Parameters in a response message body	32

Table 2-11 bind_transceiver error codes	33

Table 2-12 Parameters in a enquire_link request message header	34

Table 2-13 Parameters in a enquire_link response message header	35

Table 2-14 enquire_link error codes	36

Table 2-15 Parameters in a deliver_sm request message header	37

Table 2-16 Parameters in a deliver_sm request message body	38

Table 2-17 Parameter structure of the source_port type	55

Table 2-18 Parameter structure of the destination_port type	56

Table 2-19 Parameter structure of the sar_msg_ref_num type	56

Table 2-20 Parameter structure of the sar_total_segments type	56

Table 2-21 Parameter structure of the sar_segment_seqnum type	57

Table 2-22 Parameter structure of the payload_type type	58

Table 2-23 Parameter structure of the message_payload type	59

Table 2-24 Parameter structure of the receipted_message_id type	60

Table 2-25 Parameters in a deliver_sm response message header	60

Table 2-26 Parameters in a response message body	61

Table 2-27 Parameters in a unbind request message header	62

Table 2-28 Parameters in a unbind response message header	63

Table 2-29 unbind error codes	64

Table 3-1 Description for the process of sending SMS messages and receiving status reports	66

Table 3-2 Parameters in a bind_transmitter request message header	68

Table 3-3 Parameters in a bind_transmitter request message body	69

Table 3-4 Parameters in a bind_transmitter response message header	72

Table 3-5 Parameters in a response message body	73

Table 3-6 bind_transmitter error codes	74

Table 3-7 Parameters in a submit_sm request message header	75

Table 3-8 Parameters in a submit_sm request message body	76

Table 3-9 Parameter structure of the source_port type	101

Table 3-10 Parameter structure of the destination_port type	101

Table 3-11 Parameter structure of the sar_msg_ref_num type	102

Table 3-12 Parameter structure of the sar_total_segments type	102

Table 3-13 Parameter structure of the sar_segment_seqnum type	103

Table 3-14 Parameter structure of the payload_type type	103

Table 3-15 Parameter structure of the message_payload type	105

Table 3-16 Parameter structure of the serviceId type	105

Table 3-17 Parameter structure of the link_id type	106

Table 3-18 Parameter structure of the presentID type	106

Table 3-19 Parameters in a submit_sm response message header	107

Table 3-20 Parameters in a response message body	108

Table 3-21 submit_sm error codes	108

Table 3-22 Parameters in a submit_multi request message header	109

Table 3-23 Parameters in a submit_multi request message body	111

Table 3-24 Parameter structure of the dest_address type	134

Table 3-25 Parameter structure of the SME_dest_address type#table	135

Table 3-26 Parameter structure of the source_port type	137

Table 3-27 Parameter structure of the destination_port type	138

Table 3-28 Parameter structure of the sar_msg_ref_num type	138

Table 3-29 Parameter structure of the sar_total_segments type	139

Table 3-30 Parameter structure of the sar_segment_seqnum type	139

Table 3-31 Parameter structure of the payload_type type	140

Table 3-32 Parameter structure of the message_payload type	141

Table 3-33 Parameter structure of the serviceId type	142

Table 3-34 Parameter structure of the link_id type	142

Table 3-35 Parameter structure of the presentID type	143

Table 3-36 Parameters in a submit_multi response message header	143

Table 3-37 Parameters in a response message body	144

Table 3-38 submit_multi error codes	145

Table 3-39 Parameters in a deliver_sm(report) request message header	146

Table 3-40 Parameters in a deliver_sm request message body	147

Table 3-41 Parameter structure of the source_port type	165

Table 3-42 Parameter structure of the destination_port type	165

Table 3-43 Parameter structure of the sar_msg_ref_num type	166

Table 3-44 Parameter structure of the sar_total_segments type	166

Table 3-45 Parameter structure of the sar_segment_seqnum type	167

Table 3-46 Parameter structure of the payload_type type	168

Table 3-47 Parameter structure of the message_payload type	169

Table 3-48 Parameter structure of the receipted_message_id type	170

Table 3-49 Parameter structure of the short_message type	170

Table 3-50 Parameters in a deliver_sm(report) response message header	173

Table 3-51 Parameters in a response message body	174

Table 3-52 Parameters in a unbind request message header	175

Table 3-53 Parameters in a unbind response message header	176

Table 3-54 unbind error codes	177

Overview

  - API Functions

Third-party applications (App for short) connect to UAG and use its SMS capability to send and receive SMS messages. The App is generally developed by various partners of the UAG.

Partners are the enterprises and individuals who sign a contract and cooperate with carriers in utilizing the UAG. Partners include SPs, CPs, Developers, and Enterprises. In this document, partners are mainly the SPs, Developers, and Enterprises who use APIs for secondary development.

Table 1  describes functions of SMS capability APIs provided by the UAG.

Functions of SMS capability APIs

| Function | Subfunction | Description | API |
| --- | --- | --- | --- |
| Receiving SMS messages | Receiving SMS messages in Notify mode | The App (functioning as the client) invokes the bind_receiver API to create a unidirectional message receiving link with the UAG or invokes the bind_transceiver API to create bidirectional link with the UAG. After the link is created, the App (functioning as the client) invokes the enquire_link API to send heartbeat messages to the UAG (functioning as the server) to maintain the link. When the UAG receives a MO SMS message from a user, the UAG (functioning as the client) invokes the deliver_sm API to send the SMS message to the App (functioning as the server). The App (functioning as the client) invokes the unbind API to disconnect from the UAG (functioning as the server). | 2.2 bind_receiver 2.3 bind_transceiver 2.4 enquire_link 2.5 deliver_sm 2.6 unbind |
| Sending SMS messages | Sending SMS messages to users and receiving status reports in Notify mode | The App (functioning as the client) invokes the bind_transmitter API to create a unidirectional message sending link with the UAG or invokes the bind_transceiver API to create bidirectional link with the UAG. After the link is created, the App (functioning as the client) invokes the enquire_link API to send heartbeat messages to the UAG (functioning as the server) to maintain the link. The App (functioning as the client) invokes the submit_sm API to send single SMS messages or the submit_multi API to send bulk SMS messages to users through the UAG (functioning as the server). When receiving a status report from the SMSC, the UAG (functioning as the client) then invokes the deliver_sm(report) API to send the status report to the App (functioning as the server). The App (functioning as the client) invokes the unbind API to disconnect from the UAG (functioning as the server). | 3.2 bind_transmitter 2.3 bind_transceiver 2.4 enquire_link 3.3 submit_sm 3.4 submit_multi 3.5 deliver_sm(report) 3.6 unbind |

Level of Requirement for Parameters

The App must develop APIs based on the level of requirement for each parameter.

Level of requirement for parameters

| Type | Description |
| --- | --- |
| Mandatory | A parameter is always mandatory in a request. Parameters with the Mandatory requirement are used for access authentication or service processing. If a parameter with the Mandatory requirement is left empty in a request, access authentication or service processing fails and the request fails. |
| Conditional | A parameter is mandatory or optional in specified conditions. Parameters with the Conditional requirement are used for access authentication or service processing in specified conditions. If the specified conditions are met but a parameter with the Conditional requirement is left empty in a request, access authentication or service processing fails and the request fails. |
| Optional | A parameter is always optional. Parameters with the Optional requirement are not used for service processing. |

API Parameter Types

The App must develop APIs based on the type required for each parameter.

Table 1  describes the Short Message Peer to Peer (SMPP) API parameter types.

API parameter types

| Type | Description |
| --- | --- |
| Integer | An unsigned value with the defined number of octets. The octets will always be transmitted MSB first (Big Endian). Where reference is made to NULL settings of Integer fields, this implies that the field is zero filled. |
| C-Octet String | A series of ASCII characters terminated with the NULL character. Reference made to NULL settings of Octet-String fields implies that the field consists of a single NULL character, for example, an octet encoded with value 0x00 (zero). In the case of C-Octet String formats, the maximum field size is shown as a combination of string length and the NULL terminator, for example, an 8-character C-Octet String is encoded in 9 octets when the NULL terminator is included. |
| C-Octet String (Decimal) | A series of ASCII characters, each character representing a decimal digit (0 - 9) and terminated with the NULL character. |
| C-Octet String(Hex) | A series of ASCII characters, each character representing a hexadecimal digit (0 - F) and terminated with the NULL character. |
| Octet String | A series of octets, not necessarily NULL terminated. |

Table 2  describes the length of parameters in different types.

Parameter length

| Length | Type | Description |
| --- | --- | --- |
| 4 | Integer | Fixed size integer field. In this example the integer is of size 32 bits (4 octets). |
| Var Max 16 | C-Octet String | This string is of variable length from 1-15 ASCII characters, followed by an octet containing the NULL terminator. An empty string is encoded as a single octet containing the NULL character (0x00). |
| Fixed 1 or 17 | C-Octet String | This string has two possible lengths:- 1 octet containing the NULL character or a fixed number of characters terminated with the NULL character (in this example 16 characters plus the NULL character). |
| Var 0 - 254 | Octet String | Variable size octet string field. In this example the size of the octet string field can vary from 0 to 254 octets. |

SMPP PDU Format

The SMPP protocol is used to exchange request and response packet data units (PDUs) between SPs and the UAG on the TCP/IP network. Each SMPP operation involves a request PDU and a response PDU.

A request PDU and a response PDU have the same format (PDU header and PDU body). The PDU header is mandatory and the PDU body is optional. Table 1  describes the PDU format.

SMPP PDU format overview

| SMPP PDU |  |  |  |  |
| --- | --- | --- | --- | --- |
| PDU Header (mandatory) |  |  |  | PDU Body (Optional) |
| command length | command id | command status | sequence number | PDU Body |
| 4 octets | Length = (Command Length value - 4) octets |  |  |  |

The PDU header consists of the command length, command id, command status, and sequence number parameters. The PDU body contains mandatory parameters and optional parameters. Table 2  describes the PDU format.

SMPP PDU format description

| PDU Header/Body | SMPP PDU Field | Length | Type | Level of Requirement | Description |
| --- | --- | --- | --- | --- | --- |
| Header | command_length | 4 | Integer | Mandatory | Define the total octet length of the SMPP PDU packet including the length field. [Example] 1024 |
|  | command_id | 4 | Integer | Mandatory | The command_id field identifies the particular SMPP PDU, for example, submit_sm, deliver_sm. A unique command identifier is allocated to each SMPP request PDU in the range: 0x00000000 to 0x000001FF . A unique command identifier is also allocated to each SMPP response PDU in the range: 0x80000000 to 0x800001FF. NOTE An SMPP response command_id is identical to the corresponding request SMPP command_id, but with bit 31 set. [Example] 0x80000001 |
|  | command_status | 4 | Integer | Mandatory | The command_status field indicates the success or failure of an SMPP request. It is relevant only in the SMPP response PDU and it must contain a NULL value in an SMPP request PDU. [Example] 0x00000000 |
|  | sequence_number | 4 | Integer | Mandatory | This field contains a sequence number which allows SMPP requests and responses to be associated for correlation purposes. The use of sequence numbers for message correlation allows SMPP PDUs to be exchanged asynchronously. Assignment of the sequence_number is the responsibility of the SMPP PDU originator. The sequence_number should be increased monotonically for each submitted SMPP request PDU and must be preserved in the associated SMPP response PDU. The sequence_number may range from: 0x00000001 to 0x7FFFFFFF. [Example] 0x00000001 |
| Body | Mandatory Parameters | var. | mixed | Mandatory | A list of mandatory parameters corresponding to that SMPP PDU defined in the command_id field. For details, see the description of each parameter. |
|  | Optional Parameters | var. | mixed | Optional | A list of Optional Parameters corresponding to that SMPP PDU defined in the command_id field and included as required. For details, see the description of each parameter. |
|  | Conditional Parameters | var. | mixed | Conditional | A list of conditional parameters corresponding to that SMPP PDU defined in the command_id field. For details, see the description of each parameter. |

When Apps invoke UAG APIs, requests cannot contain the following XML characters: & > < ' "

If the preceding characters are really required, you must add escape characters before such characters. Otherwise, UAG APIs will fail to be invoked.

Mapping Between SMPP PDUs and APIs

Table 1  describes the mapping between SMPP PDUs and APIs.

Mapping between PDUs and APIs#table

| API | Request PDU | Response PDU |
| --- | --- | --- |
| bind_transmitter | bind_transmitter | bind_transmitter_resp |
| bind_receiver | bind_receiver | bind_receiver_resp |
| bind_transceiver | bind_transceiver | bind_transceiver_resp |
| enquire_link | enquire_link | enquire_link_resp |
| generic_nack | - | generic_nack |
| submit_sm | submit_sm | submit_sm_resp |
| submit_multi | submit_multi | submit_multi_resp |
| deliver_sm | deliver_sm | deliver_sm_resp |
| unbind | unbind | unbind_resp |

Optional Parameter Format

Optional Parameters are fields, which may be optionally included in an SMPP message. Optional Parameters must always appear at the end of a message, in the Optional Parameters section or Conditional Parameters section of the SMPP PDU. However, they may be included in any convenient order within the Optional Parameters section or Conditional Parameters section of the SMPP PDU and need not be encoded in the order presented in this document.

All SMPP optional parameters have a 16 bit Parameter Tag Identifier.

All optional parameters have the following general TLV (Tag, Length, Value) format. The definition of the Tag, Length and Value for each optional parameter is defined in the Table 1 .

SMPP PDU Format Description

| Parameter | Size | Type | Description |
| --- | --- | --- | --- |
| Tag | 2 | Integer | The Tag field is used to uniquely identify the particular optional parameter in question. The optional parameter Tag field is always 2 octets in length. [Example] 0x4004 |
| Length | 2 | Integer | The Length field indicates the length of the Value field in octets. The optional parameter Length field is always 2 octets in length. NOTE The length does not include the length of the Tag and Length fields. [Example] 0x0005 |
| Value | variable | variable | The Value field contains the actual data for the optional parameter in question. [Example] 0x826299E01 |

Table 2  show the SMPP supported Optional Parameters or Conditional Parameters and their associated Tag Values.

Optional or Conditional Parameter Tag values

| Tag | Value | Wireless Network Technology | Remarks |
| --- | --- | --- | --- |
| source_port | 0x020A | Generic | N/A |
| destination_port | 0x020B | Generic | N/A |
| sar_msg_ref_num | 0x020C | Generic | N/A |
| sar_total_segments | 0x020E | Generic | N/A |
| sar_segment_seqnum | 0x020F | Generic | N/A |
| payload_type | 0x0019 | Generic | N/A |
| network_error_code | 0x0423 | Generic | This parameter is supported in submit_resp,submit_multi_resp,deliver_resp,and data_resp messages in synchronous mode,and deliver_sm message in both synchronous mode and asynchronous mode. |
| message_payload | 0x0424 | Generic | N/A |
| receipted_message_id | 0x001E | Generic | N/A |
| link_id | 0x400D | Generic | N/A |
| presentID | 0x400E | Generic | N/A |
| ServiceID | 0x4004 | Generic | N/A |

Generic optional parameters may be applicable to all wireless network technologies i.e., GSM/ iDEN, TDMA and CDMA.

Error Response Format

This is a generic negative acknowledgement to an SMPP PDU submitted with an invalid message header. A generic_nack response is returned in the following cases:

Invalid command_length

If the receiving SMPP entity, on decoding an SMPP PDU, detects an invalid command_length (either too short or too long), it should assume that the data is corrupt. In such cases a generic_nack PDU must be returned to the message originator.

Unknown command_id

If an unknown or invalid command_id is received, a generic_nack PDU must also be returned to the originator.

Table 1  describes the format of the SMPP generic_nack PDU. It comprises the SMPP message header only.

GENERIC_NACK PDU Format Description

| PDU Header/Body | SMPP PDU Field | Size octets | Type | Level of Requirement | Description |
| --- | --- | --- | --- | --- | --- |
| Header | command_length | 4 | Integer | Mandatory | Define the total octet length of the SMPP PDU packet including the length field. [Example] 1024 |
|  | command_id | 4 | Integer | Mandatory | Value corresponding to generic_nack PDU. set it to 0x80000000. [Example] 0x80000000 |
|  | command_status | 4 | Integer | Mandatory | Error code corresponding to reason for sending the generic_nack. [Example] 0x00000000 |
|  | sequence_number | 4 | Integer | Mandatory | Set to sequence number of original PDU or to NULL if the original PDU cannot be decoded. [Example] 0x00000001 |

Table 2  describes SMPP34 return codes.

SMPP34 return codes#table

| Return codes | Value | Description |
| --- | --- | --- |
| ESME_ROK | 0x00000000 | No Error |
| ESME_RINVMSGLEN | 0x00000001 | Message Length is invalid |
| ESME_RINVCMDLEN | 0x00000002 | Command Length is invalid |
| ESME_RINVCMDID | 0x00000003 | Invalid Command ID |
| ESME_RINVBNDSTS | x00000004 | Incorrect BIND Status for given command |
| ESME_RALYBND | 0x00000005 | ESME Already in Bound State |
| ESME_RINVPRTFLG | 0x00000006 | Invalid Priority Flag |
| ESME_RINVREGDLVFLG | 0x00000007 | Invalid Registered Delivery Flag |
| ESME_RSYSERR | 0x00000008 | System Error |
| Reserved | 0x00000009 | Reserved |
| ESME_RINVSRCADR | 0x0000000A | Invalid Source Address |
| ESME_RINVDSTADR | 0x0000000B | Invalid Dest Addr |
| ESME_RINVMSGID | 0x0000000C | Message ID is invalid |
| ESME_RBINDFAIL | 0x0000000D | 0x0000000D Bind Failed |
| ESME_RINVPASWD | 0x0000000E | Invalid Password |
| ESME_RINVSYSID | 0x0000000F | Invalid System ID |
| Reserved | 0x00000010 | Reserved |
| ESME_RCANCELFAIL | 0x00000011 | Cancel SM Failed |
| Reserved | 0x00000012 | Reserved |
| ESME_RREPLACEFAIL | 0x00000013 | Replace SM Failed |
| ESME_RMSGQFUL | 0x00000014 | Message Queue Full |
| ESME_RINVSERTYP | 0x00000015 | Invalid Service Type |
| Reserved | 0x00000016-0x00000032 | Reserved |
| ESME_RINVNUMDESTS | 0x00000033 | Invalid number of destinations |
| ESME_RINVDLNAME | 0x00000034 | Invalid Distribution List name |
| Reserved | 0x00000035-0x0000003F | Reserved |
| ESME_RINVDESTFLAG | 0x00000040 | Destination flag is invalid (submit_multi) |
| Reserved | 0x00000041 | Reserved |
| ESME_RINVSUBREP | 0x00000042 | Invalid submit with replace request (i.e. submit_sm with replace_if_present_flag set) |
| ESME_RINVESMCLASS | 0x00000043 | Invalid esm_class field data |
| ESME_RCNTSUBDL | 0x00000044 | Cannot Submit to Distribution List |
| ESME_RSUBMITFAIL | 0x00000045 | submit_sm or submit_multi failed |
| Reserved | 0x00000046- 0x00000047 | Reserved |
| ESME_RINVSRCTON | 0x00000048 | Invalid Source address TON |
| ESME_RINVSRCNPI | 0x00000049 | Invalid Source address NPI |
| ESME_RINVDSTTON | 0x00000050 | Invalid Destination address TON |
| ESME_RINVDSTNPI | 0x00000051 | Invalid Destination address NPI |
| Reserved | 0x00000052 | Reserved |
| ESME_RINVSYSTYP | 0x00000053 | Invalid system_type field |
| ESME_RINVREPFLAG | 0x00000054 | Invalid replace_if_present flag |
| ESME_RINVNUMMSGS | 0x00000055 | Invalid number of messages |
| Reserved | 0x00000056- 0x00000057 | Reserved |
| ESME_RTHROTTLED | 0x00000058 | Throttling error (ESME has exceeded allowed message limits) |
| Reserved | 0x00000059- 0x00000060 | Reserved |
| ESME_RINVSCHED | 0x00000061 | Invalid Scheduled Delivery Time |
| ESME_RINVEXPIRY | 0x00000062 | Invalid message validity period (Expiry time) |
| ESME_RINVDFTMSGID | 0x00000063 | Predefined Message Invalid or Not Found |
| ESME_RX_T_APPN | 0x00000064 | ESME Receiver Temporary App Error Code |
| ESME_RX_P_APPN | 0x00000065 | ESME Receiver Permanent App Error Code |
| ESME_RX_R_APPN | 0x00000066 | ESME Receiver Reject Message Error Code |
| Reserved | 0x00000068 - 0x000000BF | Reserved |
| ESME_RINVOPTPARSTREAM | 0x000000C0 | Error in the optional part of the PDU Body. |
| ESME_ROPTPARNOTALLWD | 0x000000C1 | Optional Parameter not allowed |
| ESME_RINVPARLEN | 0x000000C2 | Invalid Parameter Length. |
| ESME_RMISSINGOPTPARAM | 0x000000C3 | Expected Optional Parameter missing |
| ESME_RINVOPTPARAMVAL | 0x000000C4 | Invalid Optional Parameter Value |
| Reserved | 0x000000C5 - 0x000000FD | Reserved |
| ESME_RUNKNOWNERR | 0x000000FF | Unknown Error |
| Reserved for SMPP extension | 0x00000100- 0x000003FF | Reserved for SMPP extension |
| Reserved | 0x00000500- 0xFFFFFFFF | Reserved |
| Reserved for SMSC vendor specific errors | 0x00000400- 0x000004FF | Reserved for SMSC vendor specific errors |

APIs for Receiving SMS Messages

  - Process

The process of the App receiving SMS messages consists of the following main steps:

Creating a message receiving link: The App creates a message receiving link with the UAG.

Maintaining the link: After the link is created, the App sends heartbeat messages to the UAG to maintain the link.

Receiving MO SMS messages: The App receives MO SMS messages from the UAG in real time.

Disconnecting the link: The App disconnects from the UAG.

Figure 1  shows the process of receiving SMS messages in Notify mode.

Process of receiving SMS messages in Notify mode#fig

Table 1  describes the process.

Description for the process of receiving SMS messages

| Step | Description |
| --- | --- |
| 1-2 | The App sends a request to the UAG to create a link. The UAG performs authentication and service level agreement (SLA) control based on fields in the request, saves MO routing information in the database, and sends a response to the App. |
| 3-4 | The App sends heartbeat messages to the UAG to maintain the link. The UAG sends responses to the App. |
| 5-6 | The UAG receives an MO SMS message from a user, matches an MO route based on the access code, and sends a notification of the SMS message to the App. The App parses the notification and sends a response to the UAG. |
| 7-8 | The App sends the unbind request to the UAG to disconnecting the link. The UAG sends responses to the App. |

bind_receiver

    - Function

The App (functioning as the client) invokes the bind_receiver API to establish a network connection with the UAG (functioning as the server) to receive messages. The UAG send a response within 60 seconds by default.

After the network connection is established, the UAG sends MO SMS messages received from users to the App based on the network connection.

Partners must code the App based on the API field requirements so that the App can send correct requests to the UAG.

Request

The App functions as the client and sends a bind_receiver request message to the UAG to establish a network connection for receiving messages.

Message Header Parameters

Table 1  describes parameters in a bind_receiver request message header.

Parameters in a bind_receiver request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the bind_receiver PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to bind_receiver request. Set it to 0x00000001. [Example] 0x00000001 |
| command_status | Integer | 4 | Mandatory | Not used in bind_receiver PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set to a unique sequence number. The associated bind_receiver_resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a bind_receiver request message body.

Parameters in a bind_receiver request message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| system_id | C-Octet String | Var. max 16 | Mandatory | Partner ID. The ID is automatically allocated by the UAG to partners after successful registration. To obtain the ID: An SP can log in to the UAG management portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. A Developer can log in to the Developer Portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. An Enterprise must contact the carrier. [Example] 0xC9 |
| password | C-Octet String | Var. max 9 | Mandatory | Authentication key for the UAG to authenticate partners. To obtain the ID: An SP can obtain the password from the email notification received after successful registration. A Developer can log in to the Developer Portal, choose Member Center > Account > Registration Information > Invoke Password, and set the password. An Enterprise must contact the carrier to obtain the password. |
| system_type | C-Octet String | Var. max 13 | Mandatory | Identifies the type of App requesting to bind as a receiver with the UAG. Set it to NULL. [Example] 0x00 |
| interface_version | Integer | 1 | Mandatory | The version of the SMPP protocol. 0x00-0x33: SMPP version 3.3 0x34: SMPP version 3.4 [Example] 0x34 |
| addr_ton | Integer | 1 | Mandatory | Type of Number (TON) for App address(es) served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |
| addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator (NPI) for App address(es) served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |
| address_range | C-Octet String | Var. max 41 | Mandatory | A single App address or a range of App addresses served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |

Response

The UAG functions as the server, processes bind_receiver request messages received from the App, and sends bind_receiver response messages to the App.

This topic provides a success response example. If a request fails, the UAG sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a bind_receiver response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the bind_receiver_resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to bind_receiver_resp. Set it to 0x80000001. [Example] 0x80000001 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original bind_receiver request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original bind_receiver request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a response message body.

Parameters in a response message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| system_id | C-Octet String | Var. max 16 | Mandatory | Partner ID. The ID is automatically allocated by the UAG to partners after successful registration. To obtain the ID: An SP can log in to the UAG management portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. A Developer can log in to the Developer Portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. An Enterprise must contact the carrier. [Example] 0xC9 |

Error Codes

Table 1  describes bind_receiver error codes that the UAG may return upon an exception. For details about the error codes, see the UAG Solution Error Code Reference.

bind_receiver error codes

| Error Code | Description |
| --- | --- |
| 0x00000001 | Message Length is invalid. |
| 0x0000000D | Bind Failed. |
| 0x0000000E | Invalid Password. |

bind_transceiver

    - Function

The App (functioning as the client) invokes the API to create a bidirectional link with the UAG (functioning as the server). The UAG send a response within 60 seconds by default.

After the link is created, the App sends and receives messages using the link.

The UAG sends MO SMS messages received from users to the App using the link.

The App (functioning as the client) sends SMS messages to the UAG (functioning as the server) using the link.

Partners must code the App based on the API field requirements so that the App can send correct requests to the UAG.

Request

The App functions as the client and sends a bind_transceiver request message to the UAG to establish a network connection for receiving or sending messages.

Message Header Parameters

Table 1  describes parameters in a bind_transceiver request message header.

Parameters in a bind_transceiver request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the bind_transceiver PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to bind_transceiver request set it to 0x00000009. [Example] 0x00000009 |
| command_status | Integer | 4 | Mandatory | Not used in bind_transceiver PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Conditional | sequence number. Set to a unique sequence number. The associated bind_transceiver_resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a bind_transceiver request message body.

Parameters in a bind_transceiver request message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| system_id | C-Octet String | Var. max 16 | Mandatory | Partner ID. The ID is automatically allocated by the UAG to partners after successful registration. To obtain the ID: An SP can log in to the UAG management portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. A Developer can log in to the Developer Portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. An Enterprise must contact the carrier. [Example] 0xC9 |
| password | C-Octet String | Var. max 9 | Mandatory | Authentication key for the UAG to authenticate partners. To obtain the ID: An SP can obtain the password from the email notification received after successful registration. A Developer can log in to the Developer Portal, choose Member Center > Account > Registration Information > Invoke Password, and set the password. An Enterprise must contact the carrier to obtain the password. |
| system_type | C-Octet String | Var. max 13 | Mandatory | Identifies the type of App requesting to bind as a receiver with the UAG. Set it to NULL. [Example] 0x00 |
| interface_version | Integer | 1 | Mandatory | The version of the SMPP protocol. 0x00-0x33: SMPP version 3.3 0x34: SMPP version 3.4 [Example] 0x34 |
| addr_ton | Integer | 1 | Mandatory | Type of Number (TON) for App address(es) served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |
| addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator (NPI) for App address(es) served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |
| address_range | C-Octet String | Var. max 41 | Mandatory | A single App address or a range of App addresses served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |

Response

The UAG functions as the server, processes bind_transceiver request messages received from the App, and sends bind_transceiver response messages to the App.

This topic provides a success response example. If a request fails, the UAG sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a bind_transceiver response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the bind_transceiver_resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to bind_transceiver_resp. Set it to 0x80000009. [Example] 0x80000009 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original bind_transceiver request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original bind_transceiver request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a response message body.

Parameters in a response message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| system_id | C-Octet String | Var. max 16 | Mandatory | Partner ID. The ID is automatically allocated by the UAG to partners after successful registration. To obtain the ID: An SP can log in to the UAG management portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. A Developer can log in to the Developer Portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. An Enterprise must contact the carrier. [Example] 0xC9 |

Error Codes

Table 1  describes bind_transceiver error codes that the UAG may return upon an exception. For details about the error codes, see the UAG Solution Error Code Reference.

bind_transceiver error codes

| Error Code | Description |
| --- | --- |
| 0x00000001 | Message Length is invalid. |
| 0x0000000D | Bind Failed. |
| 0x0000000E | Invalid Password. |

enquire_link

    - Function

After the link is created, the App (functioning as the client) invokes the enquire_link API to send heartbeat messages to the UAG (functioning as the server) to maintain the link.

Partners must code the App based on the API field requirements so that the App can send correct requests to the UAG.

Request

The App (functioning as the client) invokes the enquire_link API to send heartbeat messages to the UAG (functioning as the server) to maintain the link.

Message Header Parameters

Table 1  describes parameters in a enquire_link request message header.

Parameters in a enquire_link request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the enquire_link PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to enquire_link request Set it to 0x00000015. [Example] 0x00000015 |
| command_status | Integer | 4 | Mandatory | Not used in bind_transceiver PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Conditional | sequence number. Set to a unique sequence number. The associated enquire_link_resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

None.

Response

The UAG functions as the server, processes enquire_link request messages received from the App, and sends enquire_link response messages to the App.

This topic provides a success response example. If a request fails, the UAG sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a enquire_link response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the enquire_link_resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to enquire_link_resp. Set it to 0x80000015. [Example] 0x80000015 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original enquire_link request. Set it to 0x00000000. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original enquire_link request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

None.

Error Codes

Table 1  describes enquire_link error codes that the UAG may return upon an exception. For details about the error codes, see the UAG Solution Error Code Reference.

enquire_link error codes

| Error Code | Description |
| --- | --- |
| 0x00000003 | Invalid Command ID |

deliver_sm

    - Function

The UAG (functioning as the client) invokes the deliver_sm API to send MO SMS messages to the App (functioning as the server).

After the App creates a unidirectional or bidirectional link using the 2.3 bind_transceiver or 3.2 bind_transmitter API, the UAG invokes the deliver_sm API to send SMS messages received from users to the App. If the MO SMS messages fail to be sent, the UAG resends the messages to the App when any of the cached message resending criteria is met. Cached SMS messages can be resent for a maximum of five times. SMS messages can be resent at least 1800 seconds after a sending failure.

Partners must code the App based on the API field requirements so that the App can correctly parse and respond to requests received from the UAG. The App must send a response to the UAG within 30 seconds.

Request

The UAG functions as the client and sends deliver_sm request messages to the App to delivery an MO SMS message.

Message Header Parameters

Table 1  describes parameters in a deliver_sm request message header.

Parameters in a deliver_sm request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the deliver_sm PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to deliver_sm request. Set it to 0x00000005. [Example] 0x00000005 |
| command_status | Integer | 4 | Mandatory | Not used in deliver_sm PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | Sequence number. Set to a unique sequence number. The associated deliver_sm_resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a deliver_sm request message body.

Parameters in a deliver_sm request message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| service_type | C-Octet String | Var. max 6 | Mandatory | The service_type parameter can be used to indicate the SMS Application service associated with the message. Specifying the service_type allows the App to avail of enhanced messaging services such as replace by service type to control the teleservice used on the air interface. The options are as follows: NULL: Default CMT: Cellular Messaging CPT: Cellular Paging VMN: Voice Mail Notification VMA: Voice Mail Alerting WAP: Wireless Application Protocol USSD: Unstructured Supplementary Services Data [Example] 0x00 |
| source_addr_ton | Integer | 1 | Mandatory | Type of Number for source address. The options are as follows: 00000000: Unknown 00000001: International 00000010: National 00000011: Network Specific 00000100: Subscriber Number 00000101: Alphanumeric 00000110: Abbreviated other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| source_addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator for source address. The options are as follows: 00000000: Unknown 00000001: ISDN (E163/E164) 00000011: Data (X.121) 00000100: Telex (F.69) 00000110: Land Mobile (E.212) 00001000: National 00001001: Private 00001010: ERMES 00001110: Internet (IP) 00010010: WAP Client Id (to be defined by WAP Forum) Other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| source_addr | C-Octet String | Var. max 21 | Mandatory | Mobile number of the message sender. The format of the source mobile number is determined by the source_addr_ton and source_addr_npi parameters. [Example] 0x3132333435 |
| dest_addr_ton | Integer | 1 | Mandatory | Type of Number for destination. The options are as follows: 00000000: Unknown 00000001: International 00000010: National 00000011: Network Specific 00000100: Subscriber Number 00000101: Alphanumeric 00000110: Abbreviated other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. [Example] 0x00 |
| dest_addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator for destination. The options are as follows: 00000000: Unknown 00000001: ISDN (E163/E164) 00000011: Data (X.121) 00000100: Telex (F.69) 00000110: Land Mobile (E.212) 00001000: National 00001001: Private 00001010: ERMES 00001110: Internet (IP) 00010010: WAP Client Id (to be defined by WAP Forum) Other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. [Example] 0x00 |
| destination_addr | C-Octet String | Var. max 21 | Mandatory | Address of the App which received this message. In an SP's request, the value is a service access code obtained from carriers before service release. In an Enterprise's or a Developer's request, the value is an access code allocated by carriers during capability product purchase. The message format is determined by the destination_addr_ton and destination_addr_npi parameters. [Example] 0x4132333435 |
| esm_class | Integer | 1 | Mandatory | Indicates Message Type and enhanced network services. Bits 5-2 indicates the Message Type. When the bits 2 to 5 in the esm_class field are 0000, it indicates an MO SMS message. xx0000xx: Default message Type (for example, normal message) [Example] 0x000000 |
| protocol_id | Integer | 1 | Mandatory | Protocol Identifier. Network specific field. GSM: Set according to GSM 03.40. NOTE For details, see Technical Realisation of the Short Message Service Point to Point at http://www.etsi.fr. ANSI-136 (TDMA): For mobile terminated messages, this field is not used and is therefore ignored by the SMSC. For ANSI-136 mobile originated messages, the SMSC should set this value to NULL. IS-95 (CDMA): For mobile terminated messages, this field is not used and is therefore ignored by the SMSC. For IS-95 mobile originated messages, the SMSC should set this value to NULL. [Example] 0x00 |
| priority_flag | Integer | 1 | Mandatory | Designates the priority level of the message. The options are as follows: 0x00: Level 0 (lowest) priority 0x01: Level 1 priority 0x02: Level 2 priority 0x03: Level 3 (highest) priority >0x03: Reserved The value options have different meanings on the GSM, ANSI-136, and IS-95 mobile networks. GSM: For GSM mobile terminated messages with priority greater than Level 0 are treated as priority when making a delivery attempt (for example, a delivery attempt is made even when MWD is set in the HLR). 0x00: non-priority 0x01, 0x02, 0x03: priority ANSI-136: 0x00: Bulk 0x01: Normal 0x02: Urgent 0x03: Very Urgent IS-95: 0x00: Normal 0x01: Interactive 0x02: Urgent 0x03: Emergency [Example] 0x00 |
| schedule_delivery_time | C-Octet String | 1 or 17 | Mandatory | This field is unused for deliver_sm. It must be set to NULL. [Example] 0x00 |
| validity_period | C-Octet String | 1 or 17 | Mandatory | This field is unused for deliver_sm. It must be set to NULL. [Example] 0x00 |
| registered_delivery | Integer | 1 | Mandatory | Indicates if an App acknowledgement is required. The options are as follows: xxxx00xx: No recipient SME acknowledgment requested (default). xxxx01xx: SME Delivery Acknowledgement requested. xxxx10xx: SME Manual/User Acknowledgment requested. xxxx11xx: Both Delivery and Manual/User Acknowledgment requested [Example] 0x0A |
| replace_if_present_flag | Integer | 1 | Mandatory | Reserved. Must be set it to NULL. [Example] 0x00 |
| data_coding | Integer | 1 | Mandatory | Defines the encoding scheme of the short message user data. The options are as follows: 00000000L: SMSC Default Alphabet. 00000001: IA5(CCITT T.50)/ASCII(ANSI X3.4). 00000010: Octet unspecified (8-bit binary). 00000011: Latin 1(ISO-8859-1). 00000100: Octet unspecified (8-bit binary). 00000101: JIS(X 0208-1990). 00000110: Cyrllic(ISO-8859-5). 00000111: Latin/Hebrew(ISO-8859-8). 00001000: UCS2(ISO/IEC-10646). 00001001: Pictogram Encoding. 00001010: ISO-2022-JP(Music Codes). 00001011: Reserved. 00001100: Reserved. 00001101: Extended Kanji JIS(X 0212-1990). 00001110: KS C 5601. 00001111-C10111111: Reserved. 1100xxxx: GSM MWI control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. 1101xxxx: GSM MWI control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. 11101111: Reserved. 1111xxxx: GSM message class control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. [Example] 0x00 |
| sm_default_msg_id | Integer | 1 | Mandatory | This field is unused for deliver_sm. It must be set to NULL. [Example] 0x00 |
| sm_length | Integer | 1 | Mandatory | Length in octets of the short_message field. It should be set to 0 if the message_payload parameter is being used to send user data larger than 254 octets. 0x00: No user data in short message field. 0x01-C0xFE: Allowed 0xFF: Not allowed [Example] 0x11 |
| short_message | Octet String | Var. 0-254 | Mandatory | MO SMS message content. When the bits 2 to 5 in the esm_class field are 0000, the short_message indicates an MO SMS message content. Up to 254 octets of short message user data. The exact physical limit for short_message size may vary according to the underlying network. Apps which need to send messages longer than 254 octets should use the message_payload parameter. In this case the sm_length field should be set to zero. NOTE The short message data should be inserted in either the short_message or message_payload fields. Both fields must not be used simultaneously. [Example] sms test |
| source_port | TLV | N/A | Optional | Indicates the application port number associated with the source address of the message. This parameter should be present for WAP applications. For details about the source_port type, see Table 3 . |
| destination_port | TLV | N/A | Optional | Indicates the application port number associated with the destination address of the message. This parameter should be present for WAP applications. For details about the destination_port type, see Table 4 . |
| sar_msg_ref_num | TLV | N/A | Optional | The reference number for a particular concatenated short message. This parameter shall contain a originator generated reference number so that a segmented short message may be reassembled into a single original message. This allows the parallel transmission of several segmented messages. This reference number shall remain constant for every segment which makes up a particular concatenated short message. When present, the PDU must also contain the sar_total_segments and sar_segment_seqnum parameters. Otherwise this parameter shall be ignored. For details about the sar_msg_ref_num type, see Table 5 . |
| sar_total_segments | TLV | N/A | Optional | Indicates the total number of short messages within the concatenated short message. When present, the PDU must also contain the sar_msg_ref_num and sar_segment_seqnum parameters. Otherwise this parameter shall be ignored. For details about the sar_total_segments type, see Table 6 . |
| sar_segment_seqnum | TLV | N/A | Optional | Indicates the sequence number of a particular short message fragment within the concatenated short message. When present, the PDU must also contain the sar_total_segments and sar_msg_ref_num parameters. Otherwise this parameter shall be ignored. For details about the sar_segment_seqnum type, see Table 7 . |
| payload_type | TLV | N/A | Optional | Defines the type of payload (for example, WDP and WCMP). For details about the payload_type type, see Table 8 . |
| message_payload | TLV | N/A | Optional | Contains the extended short message user data. Up to 64K octets can be transmitted. The short message data should be inserted in either the short_message or message_payload fields. Both fields should not be used simultaneously. The sm_length field should be set to zero if using the message_payload parameter. For details about the message_payload type, see Table 9 . |
| receipted_message_id | TLV | N/A | Optional | Unique ID of the MO SMS message received by the UAG. For details about the receipted_message_id type, see Table 10 . |

Table 3  describes the parameter structure of the source_port type.

Parameter structure of the source_port type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020A. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | All values allowed. [Example] 0x50 |

Table 4  describes the parameter structure of the destination_port type.

Parameter structure of the destination_port type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020B. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | All values allowed. [Example] 0x50 |

Table 5  describes the parameter structure of the sar_msg_ref_num type.

Parameter structure of the sar_msg_ref_num type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020C. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | Reference number. [Example] 0x0001 |

Table 6  describes the parameter structure of the sar_total_segments type.

Parameter structure of the sar_total_segments type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020E. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 1 | Mandatory | This parameter shall contain a value in the range 1 to 255 indicating the total number of fragments within the concatenated short message. The value shall start at 1 and remain constant for every short message which makes up the concatenated short message. [Example] 0x05 |

Table 7  describes the parameter structure of the sar_segment_seqnum type.

Parameter structure of the sar_segment_seqnum type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020F. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 1 | Mandatory | This octet shall contain a value in the range 1 to 255 indicating the sequence number of a particular message within the concatenated short message. The value shall start at 1 and increment by one for every message sent within the concatenated short message. [Example] 0x01 |

Table 8  describes the parameter structure of the payload_type type.

Parameter structure of the payload_type type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x0019. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Integer | 1 | Mandatory | 0x00: Default.In the case of a WAP application, the default higher layer message type is a WDP message. See Wireless Datagram Protocol Specification from http://www.wapforum.org for details. 0x01: WCMP message. Wireless Control Message Protocol formatted data. See Wireless Datagram Protocol Specification from http://www.wapforum.org for details. 0x02-C0xFF: Reserved [Example] 0x01 |

Table 9  describes the parameter structure of the message_payload type.

Parameter structure of the message_payload type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x0424. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | C-Octetstring | Var | Mandatory | Short message user data. The maximum size is SMSC and network implementation specific. Contact carriers to obtain the value. [Example] 0x01 |

Table 10  describes the parameter structure of the receipted_message_id type.

Parameter structure of the receipted_message_id type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x001E. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | C-Octet String | Var. max 65 | Mandatory | Message ID. [Example] 0xB3A73CE2F77 |

Response

The App functions as the server, processes deliver_sm request messages received from the UAG, and sends response messages to the UAG

This topic provides a success response example and describes parameters in the response. If a request fails, the App sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a deliver_sm response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the deliver_sm_resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to deliver_sm_resp. Set it to 0x80000005. [Example] 0x80000005 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original deliver_sm request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original deliver_sm request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a response message body.

Parameters in a response message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| message_id | C-Octet String | Var. max 65 | Mandatory | This field is unused and is set to NULL. [Example] 0x00 |

Error Codes

The App returns error codes to the UAG when an exception occurs in response to deliver_sm requests. The error codes are provided by the App based on SMPP specifications.

unbind

    - Function

The App (functioning as the client) invokes the unbind API to disconnect from the UAG (functioning as the server).

Partners must code the App based on the API field requirements so that the App can send correct requests to the UAG.

Request

The App functions as the client and sends a unbind request message to the UAG to disconnect from the UAG.

Message Header Parameters

Table 1  describes parameters in a unbind request message header.

Parameters in a unbind request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the unbind PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to unbind request. Set it to 0x00000006. [Example] 0x00000006 |
| command_status | Integer | 4 | Mandatory | Not used in unbind PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set to a unique sequence number. The associated unbind _resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

None.

Response

The UAG functions as the server, processes unbind request messages received from the App, and sends unbind response messages to the App.

This topic provides a success response example. If a request fails, the UAG sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a unbind response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the unbind _resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to unbind _resp. Set it to 0x80000006. [Example] 0x80000006 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original unbind request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original unbind request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

None.

Error Codes

Table 1  describes unbind error codes that the UAG may return upon an exception. For details about the error codes, see the UAG Solution Error Code Reference.

unbind error codes

| Error Code | Description |
| --- | --- |
| 0x00000001 | Message Length is invalid. |
| 0x00000002 | Command Length is invalid. |
| 0x00000003 | Invalid Command ID. |
| 0x00000008 | System Error. |

APIs for Sending SMS Messages

  - Process

After the App sends an SMS message to a user, the UAG sends a status report to the App in Notify mode. Figure 1  shows the process of sending SMS messages and receiving status reports in Notify mode.

Process of sending SMS messages and receiving status reports in Notify mode#fig

Table 1  describes the process.

Description for the process of sending SMS messages and receiving status reports

| Step | Description |
| --- | --- |
| 1-2 | The App sends a request to the App to create a link. The App sends a response to the App. |
| 3-4 | The App sends heartbeat messages to the UAG to maintain the link. The UAG sends responses to the App. |
| 5-6 | The App sends a request to the UAG to send an SMS message. The UAG performs authentication and SLA control based on fields in the request and sends a response to the App. Then the UAG authenticates the service, determines whether the App requires a status report based on the registered_delivery_flag field in the request, and saves the mapping between the SMS message and status report. |
| 7-8 | The UAG receives a status report and sends a notification of the status report to the App based on registered_delivery_flag. The App parses the notification and sends a response to the UAG. |
| 9-10 | The App sends the unbind request to the UAG to disconnecting the link. The UAG sends responses to the App. |

bind_transmitter

    - Function

The App (functioning as the client) invokes the bind_transmitter API to establish a network connection with the UAG (functioning as the server) to send messages.

After the network connection is established, the App can send MT SMS messages to the UAG based on the network connection.

Partners must code the App based on the API field requirements so that the App can send correct requests to the UAG. The SDP send a response within 60 seconds by default.

Request

The App functions as the client and sends a bind_transmitter request message to the UAG to establish a network connection for receiving messages.

Message Header Parameters

Table 1  describes parameters in a bind_transmitter request message header.

Parameters in a bind_transmitter request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the bind_transmitter PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to bind_transmitter request Set it to 0x00000002. [Example] 0x00000002 |
| command_status | Integer | 4 | Mandatory | Not used in bind_transmitter PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Conditional | sequence number. Set to a unique sequence number. The associated bind_transmitter_resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a bind_transmitter request message body.

Parameters in a bind_transmitter request message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| system_id | C-Octet String | Var. max 16 | Mandatory | Partner ID. The ID is automatically allocated by the UAG to partners after successful registration. To obtain the ID: An SP can log in to the UAG management portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. A Developer can log in to the Developer Portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. An Enterprise must contact the carrier. [Example] 0xC9 |
| password | C-Octet String | Var. max 9 | Mandatory | Authentication key for the UAG to authenticate partners. To obtain the ID: An SP can obtain the password from the email notification received after successful registration. A Developer can log in to the Developer Portal, choose Member Center > Account > Registration Information > Invoke Password, and set the password. An Enterprise must contact the carrier to obtain the password. |
| system_type | C-Octet String | Var. max 13 | Mandatory | Identifies the type of App requesting to bind as a receiver with the UAG. Set it to NULL. [Example] 0x00 |
| interface_version | Integer | 1 | Mandatory | The version of the SMPP protocol. 0x00-0x33: SMPP version 3.3 0x34: SMPP version 3.4 [Example] 0x34 |
| addr_ton | Integer | 1 | Mandatory | Type of Number (TON) for App address(es) served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |
| addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator (NPI) for App address(es) served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |
| address_range | C-Octet String | Var. max 41 | Mandatory | A single App address or a range of App addresses served via this SMPP receiver session. Set it to NULL. [Example] 0x00 |

Response

The UAG functions as the server, processes bind_transmitter request messages received from the App, and sends bind_transmitter response messages to the App.

This topic provides a success response example. If a request fails, the UAG sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a bind_transmitter response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the bind_transmitter_resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to bind_transmitter_resp. Set it to 0x80000002. [Example] 0x80000002 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original bind_transmitter request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original bind_transmitter request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a response message body.

Parameters in a response message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| system_id | C-Octet String | Var. max 16 | Mandatory | Partner ID. The ID is automatically allocated by the UAG to partners after successful registration. To obtain the ID: An SP can log in to the UAG management portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. A Developer can log in to the Developer Portal and query account information, or log in to the mailbox used for registration and view the email notification received after successful registration. An Enterprise must contact the carrier. [Example] 0xC9 |

Error Codes

Table 1  describes bind_transmitter error codes that the UAG may return upon an exception. For details about the error codes, see the UAG Solution Error Code Reference.

bind_transmitter error codes

| Error Code | Description |
| --- | --- |
| 0x00000001 | Message Length is invalid. |
| 0x0000000D | Bind Failed. |
| 0x0000000E | Invalid Password. |

submit_sm

    - Function

The App (functioning as the client) invokes the submit_sm API to send single SMS messages to the UAG (functioning as the server).

Partners must code the App based on the API field requirements so that the App can send correct requests to the UAG. The UAG send a response within 60 seconds by default.

Request

The App functions as the client and sends submit_sm request messages to the UAG.

Message Header Parameters

Table 1  describes parameters in a submit_sm request message header.

Parameters in a submit_sm request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the submit_sm PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to submit_sm request. Set it to 0x00000004. [Example] 0x00000004 |
| command_status | Integer | 4 | Mandatory | Not used in submit_sm PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set to a unique sequence number. The associated submit_sm_resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a submit_sm request message body.

Parameters in a submit_sm request message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| service_type | C-Octet String | Var. max 6 | Mandatory | The service_type parameter can be used to indicate the SMS Application service associated with the message. Specifying the service_type allows the App to avail of enhanced messaging services such as replace by service type to control the teleservice used on the air interface. The options are as follows: NULL: Default CMT: Cellular Messaging CPT: Cellular Paging VMN: Voice Mail Notification VMA: Voice Mail Alerting WAP: Wireless Application Protocol USSD: Unstructured Supplementary Services Data Set it to NULL for default SMSC settings. [Example] 0x00 |
| source_addr_ton | Integer | 1 | Mandatory | Type of Number for source address. The options are as follows: 00000000: Unknown 00000001: International 00000010: National 00000011: Network Specific 00000100: Subscriber Number 00000101: Alphanumeric 00000110: Abbreviated other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| source_addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator for source address. The options are as follows: 00000000: Unknown 00000001: ISDN (E163/E164) 00000011: Data (X.121) 00000100: Telex (F.69) 00000110: Land Mobile (E.212) 00001000: National 00001001: Private 00001010: ERMES 00001110: Internet (IP) 00010010: WAP Client Id (to be defined by WAP Forum) Other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| source_addr | C-Octet String | Var. max 21 | Mandatory | Address of the App which originated this message. In an SP's request, the value is a service access code obtained from carriers before service release. In an Enterprise's or a Developer's request, the value is an access code allocated by carriers during capability product purchase. [Example] 0x3132333435 |
| dest_addr_ton | Integer | 1 | Mandatory | Type of Number for destination. The options are as follows: 00000000: Unknown 00000001: International 00000010: National 00000011: Network Specific 00000100: Subscriber Number 00000101: Alphanumeric 00000110: Abbreviated other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| dest_addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator for destination. The options are as follows: 00000000: Unknown 00000001: ISDN (E163/E164) 00000011: Data (X.121) 00000100: Telex (F.69) 00000110: Land Mobile (E.212) 00001000: National 00001001: Private 00001010: ERMES 00001110: Internet (IP) 00010010: WAP Client Id (to be defined by WAP Forum) Other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| destination_addr | C-Octet String | Var. max 21 | Mandatory | Mobile number of the message recipient. The format of the destination mobile number is determined by the dest_addr_ton and dest_addr_npi parameters. [Example] 0x3132333437 |
| esm_class | Integer | 1 | Mandatory | Special message attributes (including the message mode and type) of the SMS message. The esm_class parameter is encoded as follows: bits 1-0: Messaging Mode xxxxxx00: Default SMSC Mode (for example, Store and Forward) xxxxxx01: Datagram mode xxxxxx10: Forward (for example, Transaction) mode xxxxxx11: Store and Forward mode(use to select Store and Forward mode if Default SMSC Mode is non Store and Forward) bits 5-2: Message Type xx0000xx: Default message Type (for example, normal message) xx0010xx: Short Message contains App Delivery Acknowledgement xx0100xx: Short Message contains App Manual/User Acknowledgement bits 7-6: GSM Network Specific Features 00xxxxxx: No specific features selected 01xxxxxx: UDHI Indicator (only relevant for MT short messages) 10xxxxxx: Set Reply Path (only relevant for GSM network) 11xxxxxx: Set UDHI and Reply Path (only relevant for GSM network) [Example] 0x40 |
| protocol_id | Integer | 1 | Mandatory | Protocol Identifier. Network specific field. GSM: Set according to GSM 03.40. NOTE For details, see Technical Realisation of the Short Message Service Point to Point at http://www.etsi.fr. ANSI-136 (TDMA): For mobile terminated messages, this field is not used and is therefore ignored by the SMSC. For ANSI-136 mobile originated messages, the SMSC should set this value to NULL. IS-95 (CDMA): For mobile terminated messages, this field is not used and is therefore ignored by the SMSC. For IS-95 mobile originated messages, the SMSC should set this value to NULL. [Example] 0x00 |
| priority_flag | Integer | 1 | Mandatory | Designates the priority level of the message. The options are as follows: 0x00: Level 0 (lowest) priority 0x01: Level 1 priority 0x02: Level 2 priority 0x03: Level 3 (highest) priority >0x03: Reserved The value options have different meanings on the GSM, ANSI-136, and IS-95 mobile networks. GSM: For GSM mobile terminated messages with priority greater than Level 0 are treated as priority when making a delivery attempt (for example, a delivery attempt is made even when MWD is set in the HLR). 0x00: non-priority 0x01, 0x02, 0x03: priority ANSI-136: 0x00: Bulk 0x01: Normal 0x02: Urgent 0x03: Very Urgent IS-95: 0x00: Normal 0x01: Interactive 0x02: Urgent 0x03: Emergency [Example] 0x00 |
| schedule_delivery_time | C-Octet String | 1 or 17 | Mandatory | Scheduled time at which the message delivery is first attempted. The time can be the absolute time or relative time. If it is the relative time, the SMSC calculates the time based on the system time. Set to NULL for immediate message delivery. [Format] YYMMDDhhmmsstnnp In the format: t: tenths of second (0-9) nn: Time difference in quarter hours between local time (as expressed in the first 13 octets) and UTC (Universal Time Constant) time (00-48). p: UTC wizard ID. +: Local time is in quarter hours advanced in relation to UTC time. -: Local time is in quarter hours retarded in relation to UTC time. R: Local time is relative to the current SMSC time. If the time is the relative time, set t and nn to 0. [Example] 0x4CC773455000R |
| validity_period | C-Octet String | 1 or 17 | Mandatory | Validity period of this message. The parameter indicates the SMSC expiration time, after which the message should be discarded if not delivered to the destination. The time can be the absolute time or relative time. If it is the relative time, the SMSC calculates the time based on the system time. Set it to NULL to request the SMSC default validity period. [Format] YYMMDDhhmmsstnnp In the format: t: tenths of second (0-9) nn: Time difference in quarter hours between local time (as expressed in the first 13 octets) and UTC (Universal Time Constant) time (00-48). p: UTC wizard ID. +: Local time is in quarter hours advanced in relation to UTC time. -: Local time is in quarter hours retarded in relation to UTC time. R: Local time is relative to the current SMSC time. If the time is the relative time, set t and nn to 0. [Example] 0xE1754058D8120+ |
| registered_delivery | Integer | 1 | Mandatory | Indicator to signify if an SMSC delivery receipt or an App acknowledgement is required. The options are as follows: xxxxxx00: No SMSC Delivery Receipt requested (default). xxxxxx01: SMSC Delivery Receipt requested where final delivery outcome is delivery success or failure. xxxxxx10: SMSC Delivery Receipt requested where the final delivery outcome is delivery failure. [Example] 0x01 |
| replace_if_present_flag | Integer | 1 | Mandatory | Flag indicating if submitted message should replace an existing message. The SMSC will replace an existing message provided that the source address, destination address and service_type match the same fields in the new message. The options are as follows: 0x00: Do not replace (default) 0x01: Replace 0x02-C0xFF: reserved [Example] 0x00 |
| data_coding | Integer | 1 | Mandatory | Defines the encoding scheme of the short message user data. The options are as follows: 00000000L: SMSC Default Alphabet. 00000001: IA5(CCITT T.50)/ASCII(ANSI X3.4). 00000010: Octet unspecified (8-bit binary). 00000011: Latin 1(ISO-8859-1). 00000100: Octet unspecified (8-bit binary). 00000101: JIS(X 0208-1990). 00000110: Cyrllic(ISO-8859-5). 00000111: Latin/Hebrew(ISO-8859-8). 00001000: UCS2(ISO/IEC-10646). 00001001: Pictogram Encoding. 00001010: ISO-2022-JP(Music Codes). 00001011: Reserved. 00001100: Reserved. 00001101: Extended Kanji JIS(X 0212-1990). 00001110: KS C 5601. 00001111-C10111111: Reserved. 1100xxxx: GSM MWI control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. 1101xxxx: GSM MWI control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. 11101111: Reserved. 1111xxxx: GSM message class control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. [Example] 0x00 |
| sm_default_msg_id | Integer | 1 | Mandatory | Indicates the short message to send from a list of predefined (canned) short messages stored on the SMSC. If not using an SMSC canned message, set to NULL. The options are as follows: 0x00: Reserved 0x01-C0xFE: Allowed values 0XFF: Reserved [Example] 0x00 |
| sm_length | Integer | 1 | Mandatory | Length in octets of the short_message field. It should be set to 0 if the message_payload parameter is being used to send user data larger than 254 octets. 0x00: No user data in short message field. 0x01-C0xFE: Allowed 0xFF: Not allowed [Example] 0x11 |
| short_message | Octet String | Var. 0-254 | Mandatory | Up to 254 octets of short message user data. The exact physical limit for short_message size may vary according to the underlying network. Apps which need to send messages longer than 254 octets should use the message_payload parameter. In this case the sm_length field should be set to zero. NOTE The short message data should be inserted in either the short_message or message_payload fields. Both fields must not be used simultaneously. [Example] sms test |
| source_port | TLV | - | Optional | Indicates the application port number associated with the source address of the message. This parameter should be present for WAP applications. For details about the source_port type, see Table 3 . |
| destination_port | TLV | - | Optional | Indicates the application port number associated with the destination address of the message. This parameter should be present for WAP applications. For details about the destination_port type, see Table 4 . |
| sar_msg_ref_num | TLV | - | Optional | The reference number for a particular concatenated short message. This parameter shall contain a originator generated reference number so that a segmented short message may be reassembled into a single original message. This allows the parallel transmission of several segmented messages. This reference number shall remain constant for every segment which makes up a particular concatenated short message. When present, the PDU must also contain the sar_total_segments and sar_segment_seqnum parameters. Otherwise this parameter shall be ignored. For details about the sar_msg_ref_num type, see Table 5 . |
| sar_total_segments | TLV | - | Optional | Indicates the total number of short messages within the concatenated short message. When present, the PDU must also contain the sar_msg_ref_num and sar_segment_seqnum parameters. Otherwise this parameter shall be ignored. For details about the sar_total_segments type, see Table 6 . |
| sar_segment_seqnum | TLV | - | Optional | Indicates the sequence number of a particular short message fragment within the concatenated short message. When present, the PDU must also contain the sar_total_segments and sar_msg_ref_num parameters. Otherwise this parameter shall be ignored. For details about the sar_segment_seqnum type, see Table 7 . |
| payload_type | TLV | - | Optional | Defines the type of payload (for example, WDP and WCMP). For details about the payload_type type, see Table 8 . |
| message_payload | TLV | - | Optional | Contains the extended short message user data. Up to 64K octets can be transmitted. The short message data should be inserted in either the short_message or message_payload fields. Both fields should not be used simultaneously. The sm_length field should be set to zero if using the message_payload parameter. For details about the message_payload type, see Table 9 . |
| serviceId | TLV | - | Conditional | Service ID. The ID is automatically allocated by the UAG to services after successful release. An SP can log in to the UAG management portal and query service information for the ID. This parameter is mandatory in an SP's request, and can be left empty in a Developer's or an Enterprise's request. For details about the serviceId type, see Table 10 . |
| link_id | TLV | - | Conditional | Service order ID. The ID is automatically generated by the UAG when a user orders a service in the UAG. The value of link_id is valid for 24 hours by default. This parameter is mandatory during on-demand service delivery by SMS message. The UAG sends the value to SPs as follows in different scenarios: Invokes the ServiceOnDemand API to send the value when a user orders a service on the UAG portals. Invokes the deliver_sm API to send the value when a user orders a service by sending an SMS message. For details about the link_id type, see Table 11 . |
| presentID | TLV | - | Conditional | Service gift ID. The ID is automatically generated by the UAG when a user sends a service to another user as a gift on the UAG. This parameter is mandatory in an SP's request for sending an SMS message to a gift recipient, and can be left empty in an Enterprise's or a Developer's request. The UAG invokes the assignPresentToUser API to send the value to SPs. For details about the presentID type, see Table 12 . |

Table 3  describes the parameter structure of the source_port type.

Parameter structure of the source_port type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020A. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | All values allowed. [Example] 0x50 |

Table 4  describes the parameter structure of the destination_port type.

Parameter structure of the destination_port type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020B. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | All values allowed. [Example] 0x50 |

Table 5  describes the parameter structure of the sar_msg_ref_num type.

Parameter structure of the sar_msg_ref_num type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020C. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | Reference number. [Example] 0x0001 |

Table 6  describes the parameter structure of the sar_total_segments type.

Parameter structure of the sar_total_segments type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020E. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 1 | Mandatory | This parameter shall contain a value in the range 1 to 255 indicating the total number of fragments within the concatenated short message. The value shall start at 1 and remain constant for every short message which makes up the concatenated short message. [Example] 0x05 |

Table 7  describes the parameter structure of the sar_segment_seqnum type.

Parameter structure of the sar_segment_seqnum type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020F. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 1 | Mandatory | This octet shall contain a value in the range 1 to 255 indicating the sequence number of a particular message within the concatenated short message. The value shall start at 1 and increment by one for every message sent within the concatenated short message. [Example] 0x01 |

Table 8  describes the parameter structure of the payload_type type.

Parameter structure of the payload_type type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x0019. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Integer | 1 | Mandatory | 0x00: Default.In the case of a WAP application, the default higher layer message type is a WDP message. See Wireless Datagram Protocol Specification from http://www.wapforum.org for details. 0x01: WCMP message. Wireless Control Message Protocol formatted data. See Wireless Datagram Protocol Specification from http://www.wapforum.org for details. 0x02-C0xFF: Reserved [Example] 0x01 |

Table 9  describes the parameter structure of the message_payload type.

Parameter structure of the message_payload type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x0424. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | C-Octetstring | - | Mandatory | Short message user data. The maximum size is SMSC and network implementation specific. Contact carriers to obtain the value. [Example] 0x01 |

Table 10  describes the parameter structure of the serviceId type.

Parameter structure of the serviceId type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x4004. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Octet String | Var max 21 | Mandatory | Service ID. [Example] 0x1FD512A07241 |

Table 11  describes the parameter structure of the link_id type.

Parameter structure of the link_id type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x400D. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Octet String | Var max 21 | Mandatory | Service order ID. [Example] 0xB3A73CE2F77 |

Table 12  describes the parameter structure of the presentID type.

Parameter structure of the presentID type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x400E. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Octet String | Var max 15 | Mandatory | Service gift ID. [Example] 0x1452C240CF79 |

Response

The UAG functions as the server, processes submit_sm request messages received from the App, and sends submit_sm response messages to the App.

This topic provides a success response example and describes parameters in the response. If a request fails, the UAG sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a submit_sm response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the submit_sm_resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to submit_sm_resp. Set it to 0x80000004. [Example] 0x80000004 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original submit_sm request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original submit_sm request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a response message body.

Parameters in a response message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| message_id | C-Octet String | Var. max 65 | Mandatory | Unique ID of the SMS message. The UAG will contain the parameter in the request to send the status report. This parameter specifies the mapping between a status report and an MT SMS message. [Example] 0xDE0C19E529AA592 |

Error Codes

Table 1  describes submit_sm error codes that the UAG may return upon an exception. For details about the error codes, see the UAG Solution Error Code Reference.

submit_sm error codes

| Error Code | Description |
| --- | --- |
| 0x0000000A | Invalid Source Address. |
| 0x0000000B | Invalid Dest Addr. |
| 0x00000501 | The message has been licence controled. |
| 0x00000538 | The sp %1 has not orderd the api %2 in current date. |
| 0x00000539 | The sp %1 has not orderd the SCF %2. |
| 0x0000053C | Service ID %1 is not existed! |
| 0x0000053D | The service status is configuring. |
| 0x0000053E | The service status is suspended. |
| 0x0000053F | The service status is pre-deregistered. |
| 0x00000540 | The service status is deregistered. |
| 0x00000541 | Service %1 is in blacklist! |
| 0x00000542 | The API %1 is not existed. |
| 0x00000543 | The API status is disabled. |
| 0x00000544 | The sp %1 has not ordered the service %2! |
| 0x00000545 | The service %1 has not orderd the api %2. |
| 0x00000546 | The service %1 has not orderd the SCF %2. |
| 0x00000547 | SP level gross control not pass. |
| 0x00000558 | SP level request rate control not pass. |
| 0x00000559 | System level request rate control not pass. |
| 0x0000055B | Message Length is too long. |
| 0x0000055C | Dest Address size is too large. |
| 0x0000055F | request rate control not pass, sla id is %1. |
| 0x00000560 | msgType %1 is invalid. |

submit_multi

    - Function

The App (functioning as the client) invokes the submit_sm API to send bulk SMS messages to the UAG (functioning as the server).

Partners must code the App based on the API field requirements so that the App can send correct requests to the UAG. The UAG send a response within 60 seconds by default.

Request

The App functions as the client and sends submit_multi request messages to the UAG.

Message Header Parameters

Table 1  describes parameters in a submit_multi request message header.

Parameters in a submit_multi request message header

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the submit_multi PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to submit_multi request. Set it to 0x00000021. [Example] 0x00000021 |
| command_status | Integer | 4 | Mandatory | Not used in submit_multi PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set to a unique sequence number. The associated submit_multi_resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a submit_multi request message body.

Parameters in a submit_multi request message body

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| service_type | C-Octet String | Var. max 6 | Mandatory | The service_type parameter can be used to indicate the SMS Application service associated with the message. Specifying the service_type allows the App to avail of enhanced messaging services such as replace by service type to control the teleservice used on the air interface. The options are as follows: NULL: Default CMT: Cellular Messaging CPT: Cellular Paging VMN: Voice Mail Notification VMA: Voice Mail Alerting WAP: Wireless Application Protocol USSD: Unstructured Supplementary Services Data Set it to NULL for default SMSC settings. [Example] 0x00 |
| source_addr_ton | Integer | 1 | Mandatory | Type of Number for source address. The options are as follows: 00000000: Unknown 00000001: International 00000010: National 00000011: Network Specific 00000100: Subscriber Number 00000101: Alphanumeric 00000110: Abbreviated other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| source_addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator for source address. The options are as follows: 00000000: Unknown 00000001: ISDN (E163/E164) 00000011: Data (X.121) 00000100: Telex (F.69) 00000110: Land Mobile (E.212) 00001000: National 00001001: Private 00001010: ERMES 00001110: Internet (IP) 00010010: WAP Client Id (to be defined by WAP Forum) Other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| source_addr | C-Octet String | Var. max 21 | Mandatory | Address of the App which originated this message. In an SP's request, the value is a service access code obtained from carriers before service release. In an Enterprise's or a Developer's request, the value is an access code allocated by carriers during capability product purchase. [Example] 0x457531663 |
| number_of_dests | Integer | 1 | Mandatory | Number of destination addresses. Indicates the number of dest_address structures that are to follow. A maximum of 254 destination addresses are allowed. Set it to 1 when submitting to one Distribution List. [Example] 0x11 |
| dest_address | dest_address | Var. n[2-24] See Ref. | Mandatory | Contains one or more Distribution List names. The dest_address parameter is of the dest_address type and contains multiple sub-parameters. For details about the dest_address type, see Table 3 . |
| esm_class | Integer | 1 | Mandatory | The esm_class parameter is used to indicate special message attributes associated with the short message. Indicates Message Mode and Message Type. The esm_class parameter is encoded as follows: bits 1-0: Messaging Mode xxxxxx00: Default SMSC Mode (e.g. Store and Forward) xxxxxx01: Datagram mode xxxxxx10: Forward (i.e. Transaction) mode xxxxxx11: Store and Forward mode(use to select Store and Forward mode if Default SMSC Mode is non Store and Forward) bits 5-2: Message Type xx0000xx: Default message Type (i.e. normal message) xx0010xx: Short Message contains App Delivery Acknowledgement xx0100xx: Short Message contains App Manual/User Acknowledgement bits 7-6: GSM Network Specific Features 00xxxxxx: No specific features selected 01xxxxxx: UDHI Indicator (only relevant for MT short messages) 10xxxxxx: Set Reply Path (only relevant for GSM network) 11xxxxxx: Set UDHI and Reply Path (only relevant for GSM network) [Example] 01000000 |
| protocol_id | Integer | 1 | Mandatory | Protocol Identifier. Network specific field. GSM: Set according to GSM 03.40. NOTE For details, see Technical Realisation of the Short Message Service Point to Point at http://www.etsi.fr. ANSI-136 (TDMA): For mobile terminated messages, this field is not used and is therefore ignored by the SMSC. For ANSI-136 mobile originated messages, the SMSC should set this value to NULL. IS-95 (CDMA): For mobile terminated messages, this field is not used and is therefore ignored by the SMSC. For IS-95 mobile originated messages, the SMSC should set this value to NULL. [Example] 0x00 |
| priority_flag | Integer | 1 | Mandatory | Designates the priority level of the message. The options are as follows: 0x00: Level 0 (lowest) priority 0x01: Level 1 priority 0x02: Level 2 priority 0x03: Level 3 (highest) priority >0x03: Reserved The value options have different meanings on the GSM, ANSI-136, and IS-95 mobile networks. GSM: For GSM mobile terminated messages with priority greater than Level 0 are treated as priority when making a delivery attempt (i.e. a delivery attempt is made even when MWD is set in the HLR). 0x00: non-priority 0x01, 0x02, 0x03: priority ANSI-136: 0x00: Bulk 0x01: Normal 0x02: Urgent 0x03: Very Urgent IS-95: 0x00: Normal 0x01: Interactive 0x02: Urgent 0x03: Emergency [Example] 0x00 |
| schedule_delivery_time | C-Octet String | 1 or 17 | Mandatory | This parameter specifies the scheduled time at which the message delivery should be first attempted.It can be specified in either absolute time format or relative time format. If it is the relative time, the SMSC calculates the time based on the system time. Set to NULL for immediate message delivery. [Format] YYMMDDhhmmsstnnp In the format, t: tenths of second (0-9) nn: Time difference in quarter hours between local time (as expressed in the first 13 octets) and UTC (Universal Time Constant) time (00-48). p: UTC wizard ID. +: Local time is in quarter hours advanced in relation to UTC time. -: Local time is in quarter hours retarded in relation to UTC time. R: Local time is relative to the current SMSC time. If the time is the relative time, set t and nn to 0. [Example] 0x4754892328 |
| validity_period | C-Octet String | 1 or 17 | Mandatory | The validity period of this message. The parameter indicates the SMSC expiration time, after which the message should be discarded if not delivered to the destination. It can be defined in absolute time format or relative time format. If it is the relative time, the SMSC calculates the time based on the system time. Set it to NULL to request the SMSC default validity period. [Format] YYMMDDhhmmsstnnp In the format, t: tenths of second (0-9) nn: Time difference in quarter hours between local time (as expressed in the first 13 octets) and UTC (Universal Time Constant) time (00-48). p: UTC wizard ID. +: Local time is in quarter hours advanced in relation to UTC time. -: Local time is in quarter hours retarded in relation to UTC time. R: Local time is relative to the current SMSC time. If the time is the relative time, set t and nn to 0. [Example] 0x1E754058D8120+ |
| registered_delivery | Integer | 1 | Mandatory | Indicator to signify if an SMSC delivery receipt or an App acknowledgement is required. The options are as follows: xxxxxx00: No SMSC Delivery Receipt requested (default). xxxxxx01: SMSC Delivery Receipt requested where final delivery outcome is delivery success or failure. xxxxxx10: SMSC Delivery Receipt requested where the final delivery outcome is delivery failure. [Example] 0x01 |
| replace_if_present_flag | Integer | 1 | Mandatory | Flag indicating if submitted message should replace an existing message. The SMSC will replace an existing message provided that the source address, destination address and service_type match the same fields in the new message. The options are as follows: 0x0: Don't replace (default) 0x01: Replace 0x02-C0xFF: reserved [Example] 0x00 |
| data_coding | Integer | 1 | Mandatory | Defines the encoding scheme of the short message user data. The options are as follows: 00000000L: SMSC Default Alphabet. 00000001: IA5(CCITT T.50)/ASCII(ANSI X3.4). 00000010: Octet unspecified (8-bit binary). 00000011: Latin 1(ISO-8859-1). 00000100: Octet unspecified (8-bit binary). 00000101: JIS(X 0208-1990). 00000110: Cyrllic(ISO-8859-5). 00000111: Latin/Hebrew(ISO-8859-8). 00001000: UCS2(ISO/IEC-10646). 00001001: Pictogram Encoding. 00001010: ISO-2022-JP(Music Codes). 00001011: Reserved. 00001100: Reserved. 00001101: Extended Kanji JIS(X 0212-1990). 00001110: KS C 5601. 00001111-C10111111: Reserved. 1100xxxx: GSM MWI control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. 1101xxxx: GSM MWI control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. 11101111: Reserved. 1111xxxx: GSM message class control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. [Example] 0x00 |
| sm_default_msg_id | Integer | 1 | Mandatory | Indicates the short message to send from a list of predefined ('canned') short messages stored on the SMSC. If not using an SMSC canned message, set to NULL. The options are as follows: 0x0: Reserved 0x01-C0xFE: Allowed values 0xFF: Reserved [Example] 0x00 |
| sm_length | Integer | 1 | Mandatory | Size octets in octets of the short_message field. It should be set to 0 if the message_payload parameter is being used to send user data larger than 254 octets. 0x00: No user data in short message field. 0x01-C0xFE: Allowed 0xFF: Not allowed [Example] 0x11 |
| short_message | Octet String | Var. 0-254 | Mandatory | Up to 254 octets of short message user data. The exact physical limit for short_message size may vary according to the underlying network. Apps which need to send messages longer than 254 octets should use the message_payload parameter. In this case the sm_length field should be set to zero. NOTE The short message data should be inserted in either the short_message or message_payload fields. Both fields must not be used simultaneously. [Example] sms test |
| source_port | TLV | - | Optional | Indicates the application port number associated with the source address of the message. This parameter should be present for WAP applications. For details about the source_port type, see Table 5 . |
| destination_port | TLV | - | Optional | Indicates the application port number associated with the destination address of the message. This parameter should be present for WAP applications. For details about the destination_port type, see Table 6 . |
| sar_msg_ref_num | TLV | - | Optional | The reference number for a particular concatenated short message. This parameter shall contain a originator generated reference number so that a segmented short message may be reassembled into a single original message. This allows the parallel transmission of several segmented messages.This reference number shall remain constant for every segment which makes up a particular concatenated short message. When present, the PDU must also contain the sar_total_segments and sar_segment_seqnum parameters. Otherwise this parameter shall be ignored. For details about the sar_msg_ref_num type, see Table 7 . |
| sar_total_segments | TLV | - | Optional | Indicates the total number of short messages within the concatenated short message. When present, the PDU must also contain the sar_msg_ref_num and sar_segment_seqnum parameters. Otherwise this parameter shall be ignored. For details about the sar_total_segments type, see Table 8 . |
| sar_segment_seqnum | TLV | - | Optional | Indicates the sequence number of a particular short message fragment within the concatenated short message. When present, the PDU must also contain the sar_total_segments and sar_msg_ref_num parameters. Otherwise this parameter shall be ignored. For details about the sar_segment_seqnum type, see Table 9 . |
| payload_type | TLV | - | Optional | Defines the type of payload (e.g. WDP, WCMP, etc.). For details about the payload_type type, see Table 10 . |
| message_payload | TLV | - | Optional | Contains the extended short message user data. Up to 64K octets can be transmitted. The short message data should be inserted in either the short_message or message_payload fields. Both fields should not be used simultaneously. The sm_length field should be set to zero if using the message_payload parameter. For details about the message_payload type, see Table 11 . |
| serviceId | TLV | - | Conditional | Service ID. The ID is automatically allocated by the UAG to services after successful release. An SP can log in to the UAG management portal and query service information for the ID. This parameter is mandatory in an SP's request, and can be left empty in a Developer's or an Enterprise's request. For details about the serviceId type, see Table 12 . |
| link_id | TLV | - | Conditional | Service order ID. The ID is automatically generated by the UAG when a user orders a service in the UAG. The value of link_id is valid for 24 hours by default. This parameter is mandatory during on-demand service delivery by SMS message. The UAG sends the value to SPs as follows in different scenarios: Invokes the ServiceOnDemand API to send the value when a user orders a service on the UAG portals. Invokes the deliver_sm API to send the value when a user orders a service by sending an SMS message. For details about the link_id type, see Table 13 . |
| presentID | TLV | - | Conditional | Service gift ID. The ID is automatically generated by the UAG when a user sends a service to another user as a gift on the UAG. This parameter is mandatory in an SP's request for sending an SMS message to a gift recipient, and can be left empty in an Enterprise's or a Developer's request. The UAG invokes the assignPresentToUser API to send the value to SPs. For details about the presentID type, see Table 14 . |

Table 3  describes the parameter structure of the dest_address type.

Parameter structure of the dest_address type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| dest_flag | Integer | 1 | Mandatory | Flag which will identify whether destination address is a Distribution List name or SME address. 0x01: SME Address 0x02: Distribution List Name Set it to 1. [Example] 0x01 |
| Address | SME_dest_address | - | Mandatory | The Address parameter is of the SME_dest_address type and contains multiple sub-parameters. For details about the SME_dest_address type, see Table 4 . |

Table 4  describes the parameter structure of the SME_dest_address type.

Parameter structure of the SME_dest_address type#table

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| dest_addr_ton | Integer | 1 | Mandatory | Type of Number for destination. The options are as follows: 00000000: Unknown 00000001: International 00000010: National 00000011: Network Specific 00000100: Subscriber Number 00000101: Alphanumeric 00000110: Abbreviated other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| dest_addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator for destination. The options are as follows: 00000000: Unknown 00000001: ISDN (E163/E164) 00000011: Data (X.121) 00000100: Telex (F.69) 00000110: Land Mobile (E.212) 00001000: National 00001001: Private 00001010: ERMES 00001110: Internet (IP) 00010010: WAP Client Id (to be defined by WAP Forum) Other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| destination_addr | C-Octet String | Var. max 21 | Mandatory | Mobile number of the message recipient. The format of the destination mobile number is determined by the dest_addr_ton and dest_addr_npi parameters. [Example] 0x74586117 |

Table 5  describes the parameter structure of the source_port type.

Parameter structure of the source_port type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020A. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | All values allowed. [Example] 0x50 |

Table 6  describes the parameter structure of the destination_port type.

Parameter structure of the destination_port type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020B. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | All values allowed. [Example] 0x50 |

Table 7  describes the parameter structure of the sar_msg_ref_num type.

Parameter structure of the sar_msg_ref_num type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020C. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | Reference number. [Example] 0x0001 |

Table 8  describes the parameter structure of the sar_total_segments type.

Parameter structure of the sar_total_segments type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020E. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 1 | Mandatory | This parameter shall contain a value in the range 1 to 255 indicating the total number of fragments within the concatenated short message. The value shall start at 1 and remain constant for every short message which makes up the concatenated short message. [Example] 0x05 |

Table 9  describes the parameter structure of the sar_segment_seqnum type.

Parameter structure of the sar_segment_seqnum type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020F. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 1 | Mandatory | This octet shall contain a value in the range 1 to 255 indicating the sequence number of a particular message within the concatenated short message. The value shall start at 1 and increment by one for every message sent within the concatenated short message. [Example] 0x01 |

Table 10  describes the parameter structure of the payload_type type.

Parameter structure of the payload_type type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x0019. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Integer | 1 | Mandatory | 0x00: Default.In the case of a WAP application, the default higher layer message type is a WDP message. See Wireless Datagram Protocol Specification from http://www.wapforum.org for details. 0x01: WCMP message. Wireless Control Message Protocol formatted data. See Wireless Datagram Protocol Specification from http://www.wapforum.org for details. 0x02-C0xFF: Reserved [Example] 0x01 |

Table 11  describes the parameter structure of the message_payload type.

Parameter structure of the message_payload type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x0424. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | C-Octetstring | - | Mandatory | Short message user data. The maximum size is SMSC and network implementation specific. Contact carriers to obtain the value. [Example] 0x01 |

Table 12  describes the parameter structure of the serviceId type.

Parameter structure of the serviceId type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x4004. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Octet String | Var max 21 | Mandatory | Service ID. [Example] 0x1FD512A07241 |

Table 13  describes the parameter structure of the link_id type.

Parameter structure of the link_id type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x400D. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Octet String | Var max 21 | Mandatory | Service order ID. [Example] 0xB3A73CE2F77 |

Table 14  describes the parameter structure of the presentID type.

Parameter structure of the presentID type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x400E. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Octet String | Var max 15 | Mandatory | Service gift ID. [Example] 0x1452C240CF79 |

Response

The UAG functions as the server, processes submit_multi request messages received from the App, and sends submit_multi response messages to the App.

This topic provides a success response example and describes parameters in the response. If a request fails, the UAG sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a submit_multi response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the submit_multi_resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to submit_multi_resp. Set it to 0x80000021. [Example] 0x80000021 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original submit_multi request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original submit_multi request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a response message body.

Parameters in a response message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| message_id | C-Octet String | Var. max 65 | Mandatory | Unique ID of the SMS message. The UAG will contain the parameter in the request to send the status report. This parameter specifies the mapping between a status report and an MT SMS message. [Example] 0xDE0C19E529AA592 |

Error Codes

Table 1  describes submit_multi error codes that the UAG may return upon an exception. For details about the error codes, see the UAG Solution Error Code Reference.

submit_multi error codes

| Error Code | Description |
| --- | --- |
| 0x0000000A | Invalid Source Address. |
| 0x0000000B | Invalid Dest Addr. |
| 0x00000048 | Invalid Source address TON. |
| 0x00000049 | Invalid Source address NPI. |
| 0x00000050 | Invalid Destination address TON. |
| 0x00000051 | Invalid Destination address NPI. |
| 0x00000501 | The message has been licence controled. |
| 0x00000538 | The sp %1 has not orderd the api %2 in current date. |
| 0x00000539 | The sp %1 has not orderd the SCF %2. |
| 0x0000053C | Service ID %1 is not existed! |
| 0x0000053D | The service status is configuring. |
| 0x0000053E | The service status is suspended. |
| 0x0000053F | The service status is pre-deregistered. |
| 0x00000540 | The service status is deregistered. |
| 0x00000541 | Service %1 is in blacklist! |
| 0x00000542 | The API %1 is not existed. |
| 0x00000543 | The API status is disabled. |
| 0x00000544 | The sp %1 has not ordered the service %2! |
| 0x00000545 | The service %1 has not orderd the api %2. |
| 0x00000546 | The service %1 has not orderd the SCF %2. |
| 0x00000547 | SP level gross control not pass. |
| 0x00000558 | SP level request rate control not pass. |
| 0x00000559 | System level request rate control not pass. |
| 0x0000055B | Message Length is too long. |
| 0x0000055C | Dest Address size is too large. |
| 0x0000055F | request rate control not pass, sla id is %1. |
| 0x00000560 | msgType %1 is invalid. |

deliver_sm(report)

    - Function

The UAG (functioning as the client) invokes the API to send status reports to the App (functioning as the server).

When the App uses the submit_sm or submit_multi API to send SMS messages to users and the UAG successfully sends the request to the SMSC, the SMSC sends a status report to the UAG. The UAG uses the deliver_sm(report) API to report the status report to the App. If the status report fails to be reported, the UAG does not resend it.

Partners must code the App based on the API field requirements so that the App can correctly parse and respond to requests received from the UAG. The App must send a response to the UAG within 30 seconds.

Request

The UAG functions as the client and sends deliver_sm(report) request messages to the App to delivery an MO SMS message delivery status report.

Message Header Parameters

Table 1  describes parameters in a deliver_sm(report) request message header.

Parameters in a deliver_sm(report) request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the deliver_sm PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to deliver_sm request. Set it to 0x00000005. [Example] 0x00000005 |
| command_status | Integer | 4 | Mandatory | Not used in deliver_sm PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | Sequence number. Set to a unique sequence number. The associated deliver_sm_resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a deliver_sm request message body.

Parameters in a deliver_sm request message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| service_type | C-Octet String | Var. max 6 | Mandatory | The service_type parameter can be used to indicate the SMS Application service associated with the message. Specifying the service_type allows the App to avail of enhanced messaging services such as replace by service type to control the teleservice used on the air interface. The options are as follows: NULL: Default CMT: Cellular Messaging CPT: Cellular Paging VMN: Voice Mail Notification VMA: Voice Mail Alerting WAP: Wireless Application Protocol USSD: Unstructured Supplementary Services Data [Example] 0x00 |
| source_addr_ton | Integer | 1 | Mandatory | Type of Number for source address. The options are as follows: 00000000: Unknown 00000001: International 00000010: National 00000011: Network Specific 00000100: Subscriber Number 00000101: Alphanumeric 00000110: Abbreviated other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| source_addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator for source address. The options are as follows: 00000000: Unknown 00000001: ISDN (E163/E164) 00000011: Data (X.121) 00000100: Telex (F.69) 00000110: Land Mobile (E.212) 00001000: National 00001001: Private 00001010: ERMES 00001110: Internet (IP) 00010010: WAP Client Id (to be defined by WAP Forum) Other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. If not known, set it to NULL. [Example] 0x00 |
| source_addr | C-Octet String | Var. max 21 | Mandatory | Mobile number of the message sender. The format of the source mobile number is determined by the source_addr_ton and source_addr_npi parameters. [Example] 0x76514722 |
| dest_addr_ton | Integer | 1 | Mandatory | Type of Number for destination. The options are as follows: 00000000: Unknown 00000001: International 00000010: National 00000011: Network Specific 00000100: Subscriber Number 00000101: Alphanumeric 00000110: Abbreviated other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. [Example] 0x00 |
| dest_addr_npi | Integer | 1 | Mandatory | Numbering Plan Indicator for destination. The options are as follows: 00000000: Unknown 00000001: ISDN (E163/E164) 00000011: Data (X.121) 00000100: Telex (F.69) 00000110: Land Mobile (E.212) 00001000: National 00001001: Private 00001010: ERMES 00001110: Internet (IP) 00010010: WAP Client Id (to be defined by WAP Forum) Other: Reserved NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. [Example] 0x00 |
| destination_addr | C-Octet String | Var. max 21 | Mandatory | Address of the App which received this message. In an SP's request, the value is a service access code obtained from carriers before service release. In an Enterprise's or a Developer's request, the value is an access code allocated by carriers during capability product purchase. The message format is determined by the destination_addr_ton and destination_addr_npi parameters. [Example] 0x14257651 |
| esm_class | Integer | 1 | Mandatory | Indicates Message Type and enhanced network services. Bits 5-2 indicates the Message Type. When the bits 2 to 5 in the esm_class field are 0001, it indicates a status report. xx0001xx: Short Message contains SMSC Delivery Receipt [Example] 0x000100 |
| protocol_id | Integer | 1 | Mandatory | Protocol Identifier. Network specific field. GSM: Set according to GSM 03.40. NOTE For details, see Technical Realisation of the Short Message Service Point to Point at http://www.etsi.fr. ANSI-136 (TDMA): For mobile terminated messages, this field is not used and is therefore ignored by the SMSC. For ANSI-136 mobile originated messages, the SMSC should set this value to NULL. IS-95 (CDMA): For mobile terminated messages, this field is not used and is therefore ignored by the SMSC. For IS-95 mobile originated messages, the SMSC should set this value to NULL. [Example] 0x00 |
| priority_flag | Integer | 1 | Mandatory | Designates the priority level of the message. The options are as follows: 0x00: Level 0 (lowest) priority 0x01: Level 1 priority 0x02: Level 2 priority 0x03: Level 3 (highest) priority >0x03: Reserved The value options have different meanings on the GSM, ANSI-136, and IS-95 mobile networks. GSM: For GSM mobile terminated messages with priority greater than Level 0 are treated as priority when making a delivery attempt (for example, a delivery attempt is made even when MWD is set in the HLR). 0x00: non-priority 0x01, 0x02, 0x03: priority ANSI-136: 0x00: Bulk 0x01: Normal 0x02: Urgent 0x03: Very Urgent IS-95: 0x00: Normal 0x01: Interactive 0x02: Urgent 0x03: Emergency [Example] 0x00 |
| schedule_delivery_time | C-Octet String | 1 or 17 | Mandatory | This field is unused for deliver_sm. It must be set to NULL. [Example] 0x00 |
| validity_period | C-Octet String | 1 or 17 | Mandatory | This field is unused for deliver_sm. It must be set to NULL. [Example] 0x00 |
| registered_delivery | Integer | 1 | Mandatory | Indicates if an App acknowledgement is required. The options are as follows: xxxx00xx: No recipient SME acknowledgment requested (default). xxxx01xx: SME Delivery Acknowledgement requested. xxxx10xx: SME Manual/User Acknowledgment requested. xxxx11xx: Both Delivery and Manual/User Acknowledgment requested [Example] 0x0A |
| replace_if_present_flag | Integer | 1 | Mandatory | Reserved. Must be set it to NULL. [Example] 0x00 |
| data_coding | Integer | 1 | Mandatory | Defines the encoding scheme of the short message user data. The options are as follows: 00000000: SMSC Default Alphabet. 00000001: IA5(CCITT T.50)/ASCII(ANSI X3.4). 00000010: Octet unspecified (8-bit binary). 00000011: Latin 1(ISO-8859-1). 00000100: Octet unspecified (8-bit binary). 00000101: JIS(X 0208-1990). 00000110: Cyrllic(ISO-8859-5). 00000111: Latin/Hebrew(ISO-8859-8). 00001000: UCS2(ISO/IEC-10646). 00001001: Pictogram Encoding. 00001010: ISO-2022-JP(Music Codes). 00001011: Reserved. 00001100: Reserved. 00001101: Extended Kanji JIS(X 0212-1990). 00001110: KS C 5601. 00001111-C10111111: Reserved. 1100xxxx: GSM MWI control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. 1101xxxx: GSM MWI control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. 11101111: Reserved. 1111xxxx: GSM message class control. See Digital Cellular telecommunications system (Phase 2+) from http://www.etsi.fr for details. NOTE Parameter values in the value range are in binary format. The actual parameter value must be in hexadecimal format. [Example] 0x00 |
| sm_default_msg_id | Integer | 1 | Mandatory | This field is unused for deliver_sm. It must be set to NULL. [Example] 0x00 |
| sm_length | Integer | 1 | Mandatory | Length in octets of the short_message field. It should be set to 0 if the message_payload parameter is being used to send user data larger than 254 octets. 0x00: No user data in short message field. 0x01-C0xFE: Allowed 0xFF: Not allowed [Example] 0x11 |
| short_message | Octet String | Var. 0-254 | Mandatory | Status report content. When the bits 2 to 5 in the esm_class field are 0010, the short_message indicates a status report content. The typical format of a status report is id:IIIIIIIIII sub:SSS dlvrd: DDD submit date: YYMMDDhhmm done date: YYMMDDhhmm stat:DDDDDDD err: E Text:.... For details, see Table 11 . |
| source_port | TLV | N/A | Optional | Indicates the application port number associated with the source address of the message. This parameter should be present for WAP applications. For details about the source_port type, see Table 3 . |
| destination_port | TLV | N/A | Optional | Indicates the application port number associated with the destination address of the message. This parameter should be present for WAP applications. For details about the destination_port type, see Table 4 . |
| sar_msg_ref_num | TLV | N/A | Optional | The reference number for a particular concatenated short message. This parameter shall contain a originator generated reference number so that a segmented short message may be reassembled into a single original message. This allows the parallel transmission of several segmented messages. This reference number shall remain constant for every segment which makes up a particular concatenated short message. When present, the PDU must also contain the sar_total_segments and sar_segment_seqnum parameters. Otherwise this parameter shall be ignored. For details about the sar_msg_ref_num type, see Table 5 . |
| sar_total_segments | TLV | N/A | Optional | Indicates the total number of short messages within the concatenated short message. When present, the PDU must also contain the sar_msg_ref_num and sar_segment_seqnum parameters. Otherwise this parameter shall be ignored. For details about the sar_total_segments type, see Table 6 . |
| sar_segment_seqnum | TLV | N/A | Optional | Indicates the sequence number of a particular short message fragment within the concatenated short message. When present, the PDU must also contain the sar_total_segments and sar_msg_ref_num parameters. Otherwise this parameter shall be ignored. For details about the sar_segment_seqnum type, see Table 7 . |
| payload_type | TLV | N/A | Optional | Defines the type of payload (for example, WDP and WCMP). For details about the payload_type type, see Table 8 . |
| message_payload | TLV | N/A | Optional | Contains the extended short message user data. Up to 64K octets can be transmitted. The short message data should be inserted in either the short_message or message_payload fields. Both fields should not be used simultaneously. The sm_length field should be set to zero if using the message_payload parameter. For details about the message_payload type, see Table 9 . |
| receipted_message_id | TLV | N/A | Optional | Unique ID of the message sent by the UAG. Unique ID of the message delivered by the App. The value is the same as that of message_id in the submit_sm_resp or submit_multi_resp message sent by the UAG. This parameter specifies the mapping between a status report and an MT SMS message. For details about the receipted_message_id type, see Table 10 . |

Table 3  describes the parameter structure of the source_port type.

Parameter structure of the source_port type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020A. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | All values allowed. [Example] 0x50 |

Table 4  describes the parameter structure of the destination_port type.

Parameter structure of the destination_port type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020B. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | All values allowed. [Example] 0x50 |

Table 5  describes the parameter structure of the sar_msg_ref_num type.

Parameter structure of the sar_msg_ref_num type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020C. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 2 | Mandatory | Reference number. [Example] 0x0001 |

Table 6  describes the parameter structure of the sar_total_segments type.

Parameter structure of the sar_total_segments type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020E. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 1 | Mandatory | This parameter shall contain a value in the range 1 to 255 indicating the total number of fragments within the concatenated short message. The value shall start at 1 and remain constant for every short message which makes up the concatenated short message. [Example] 0x05 |

Table 7  describes the parameter structure of the sar_segment_seqnum type.

Parameter structure of the sar_segment_seqnum type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x020F. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0050 |
| Value | Integer | 1 | Mandatory | This octet shall contain a value in the range 1 to 255 indicating the sequence number of a particular message within the concatenated short message. The value shall start at 1 and increment by one for every message sent within the concatenated short message. [Example] 0x01 |

Table 8  describes the parameter structure of the payload_type type.

Parameter structure of the payload_type type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x0019. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | Integer | 1 | Mandatory | 0x00: Default.In the case of a WAP application, the default higher layer message type is a WDP message. See Wireless Datagram Protocol Specification from http://www.wapforum.org for details. 0x01: WCMP message. Wireless Control Message Protocol formatted data. See Wireless Datagram Protocol Specification from http://www.wapforum.org for details. 0x02-C0xFF: Reserved [Example] 0x01 |

Table 9  describes the parameter structure of the message_payload type.

Parameter structure of the message_payload type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x0424. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | C-Octetstring | - | Mandatory | Short message user data. The maximum size is SMSC and network implementation specific. Contact carriers to obtain the value. [Example] 0x01 |

Table 10  describes the parameter structure of the receipted_message_id type.

Parameter structure of the receipted_message_id type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| Tag | Integer | 2 | Mandatory | Set it to 0x001E. |
| Size octets | Integer | 2 | Mandatory | Size octets of value part in octets. [Example] 0x0001 |
| Value | C-Octet String | Var. max 65 | Mandatory | Message ID. [Example] 0x134734777 |

Table 11  describes the parameter structure of the short_message type.

Parameter structure of the short_message type

| Parameter | Type | Size octets | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| id | C-Octet String (Decimal) | 10 | Mandatory | Unique ID of the MT SMS message corresponding to the status report. The value is the same as that of message_id in the response sent by the UAG after the App invokes the submit_sm or submit_multi API to send SMS messages. [Example] 0x49960223 |
| sub | C-Octet String (Decimal) | 3 | Mandatory | Number of short messages originally submitted by App. The value is padded with leading zeros if necessary. [Example] 010 |
| dlvrd | C-Octet String (Decimal) | 3 | Mandatory | Number of SMS messages that the App successfully sends to the UAG. The value is padded with leading zeros if necessary. [Example] 010 |
| Submit date | C-Octet Fixed Lenth String | 10 | Mandatory | The time and date at which the short message was submitted. In the case of a message which has been replaced, this is the date that the original message was replaced. [Format] YYMMDDhhmm [Example] 0x323031343031303130313031 |
| done date | C-Octet Fixed Lenth String | 10 | Mandatory | The time and date at which the short message reached it's final state. [Format] YYMMDDhhmm [Example] 0x323031343031503130313031 |
| stat | C-Octet Fixed Lenth String | 7 | Mandatory | Status of the MT message. Enumerated values of stat: DELIVRD: Message is delivered to destination. EXPIRED: Message validity period has expired. DELETED: Message has been deleted. UNDELIV: Message is undeliverable. ACCEPTD: Message is in accepted state. UNKNOWN: Message is in invalid state. REJECTD: Message is in a rejected state. [Example] DELIVRD |
| err | C-Octet Fixed Lenth String | 3 | Mandatory | Error code. |
| text | C-Octet Fixed Lenth String | 20 | Mandatory | First 20 bytes of the original MT SMS message corresponding to the status report. [Example] sub This is a test!! |

Response

The App functions as the server, processes deliver_sm(report) request messages received from the UAG, and sends response messages to the UAG

This topic provides a success response example and describes parameters in the response. If a request fails, the App sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a deliver_sm(report) response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the deliver_sm_resp PDU in octets. [Example] 0x00000400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to deliver_sm_resp. Set it to 0x80000005. [Example] 0x80000005 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original deliver_sm request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | Sequence number. Set it to the sequence number of original deliver_sm request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

Table 2  describes parameters in a response message body.

Parameters in a response message body

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| message_id | C-Octet String | Var. max 65 | Mandatory | This field is unused and is set to NULL. [Example] 0x00 |

Error Codes

The App returns error codes to the UAG when an exception occurs in response to deliver_sm(report) request messages. The error codes are provided by the App based on SMPP specifications.

unbind

    - Function

The App (functioning as the client) invokes the unbind API to disconnect from the UAG (functioning as the server).

Partners must code the App based on the API field requirements so that the App can send correct requests to the UAG.

Request

The App functions as the client and sends a unbind request message to the UAG to disconnect from the UAG.

Message Header Parameters

Table 1  describes parameters in a unbind request message header.

Parameters in a unbind request message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the unbind PDU in octets. [Example] 0x400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to unbind request. Set it to 0x00000006. [Example] 0x00000006 |
| command_status | Integer | 4 | Mandatory | Not used in unbind PDU. Set it to NULL. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set to a unique sequence number. The associated unbind _resp PDU will echo the same sequence number. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

None.

Response

The UAG functions as the server, processes unbind request messages received from the App, and sends unbind response messages to the App.

This topic provides a success response example. If a request fails, the UAG sends an error response that contains an error code. For details about error responses, see 1.7 Error Response Format.

Message Header Parameters

Table 1  describes parameters in a response message header.

Parameters in a unbind response message header

| Parameter | Type | Length | Level of Requirement | Description |
| --- | --- | --- | --- | --- |
| command_length | Integer | 4 | Mandatory | The overall length of the unbind _resp PDU in octets. [Example] 0x400 |
| command_id | Integer | 4 | Mandatory | Value corresponding to unbind _resp. Set it to 0x80000006. [Example] 0x80000006 |
| command_status | Integer | 4 | Mandatory | Indicates status (success or error code) of original unbind request. [Example] 0x00000000 |
| sequence_number | Integer | 4 | Mandatory | sequence number. Set it to the sequence number of original unbind request. [Range] 0x00000001-0x7FFFFFFF [Example] 0x00000001 |

Message Body Parameters

None.

Error Codes

Table 1  describes unbind error codes that the UAG may return upon an exception. For details about the error codes, see the UAG Solution Error Code Reference.

unbind error codes

| Error Code | Description |
| --- | --- |
| 0x00000001 | Message Length is invalid. |
| 0x00000002 | Command Length is invalid. |
| 0x00000003 | Invalid Command ID. |
| 0x00000008 | System Error. |